Configuration Note 5004 - Ver K (2/01)

Octel 250

Message Server

Northern Telecom Meridian M-1 (DMID)

## 1.0  method of integration

The Digital Meridian Integration Device (DMID) appears as one or two digital (2616) telephones to the PBX, depending on the number of analog voice ports required.  Analog line circuits for the voice path appear on the digital set of the DMID.  When a call is received on the DMID it simultaneously rings at the analog port.  The DMID interprets call display information and transmits it to the Octel 250 which then answers the call with the appropriate personal greeting.  A separate digital set (2008) in the DMID is dedicated to set and cancel message waiting notification.

The DMID collects call integration information and transmits it to the Octel 250

## 2.0  Octel 250 ORDERING INFOrMATION

•	Digital Meridian Integration Device, which includes:
- 12- or 24-port DMID
- Power transformer
- RS-232 cable
- RS-232 integration software

Octel 250 requirements

•	Fax Line Tic (FLT) boards (8 ports/board)

•	Disk drives, based on hours of message storage needed

NOTE:  Serial channels 1, 2, and 3 in the Octel 250 are configurable as either RS232 integration links or system management terminals (SMTs). When used to support an SMT these channels require a null modem. Channel 4 is reserved for SMT use and does not require a null modem.

The following table shows the allowable combinations of integration links and SMTs:

3.0  PBX HARDWARE REQUIREMENTS

•	NT8D02 Integrated Services Digital Line (ISDL) ports:
- Revision AB or higher
- Two PBX Terminal numbers per 12-port DMID
- Three PBX Terminal numbers per 24-port DMID

PBX hardware requirements

•	Single-line ports, one per Octel 250 port, supported on either:
- NT8D03 single-line interface card
- NT8D09 single-line interface card with message waiting

•	Right-angle, 25-pair, male-amphenol cables, one per FLT

•	Two 6-wire line cords for each 12-port DMID
Three 6-wire line cords for each 24-port DMID

•	For remote service support (RDAC):
- One terminal number (TN)
- One 6-wire line cord

NOTE: Subscribers with 2500 sets with message-waiting lamps must have special line cards and power supplies to activate the message-waiting lights. If these components are not available, subscribers can use stutter dial tone for message-waiting indication.

## 3.1  PBX SOFTWARE REQUIREMENTS

•	Minimum Software:

PBX software requirements

Generic X.11, Release 15.55 – Release 25.XX:

•	BACD, Basic Automatic Call Distribution, Option 40

•	ACDA, Automatic Call Distribution, Option 45 - see section 8.2

•	MWC, Message Waiting Center, Option 46

•	EES, Enhanced End-to-End Signaling, Option 10

•	DDSP, Digit Display, Option 19

## 4.0  SUPPORTED FEATURES

•	Station forward to personal greeting
- all calls
- ring-no-answer
- busy
- do not disturb

Supported integration features

•	System forwarding to personal greeting

•	Message Waiting Indicator
- lights
- audible message waiting (stutter dial tone)

•	Automated attendant

•	Outcalling

•	Multiple return-to-operator

•	Direct call

•	Personal greeting of original-called party on double-call forward

•	Reply to message left by subscriber in internal telephone-answering mode

•	Call Sender

•	Multiple Greetings
- busy
- no answer

## 5.0  Configuring the M-1 PBX to Integrate

SPECIAL NOTE: The following represents changes to the M-1 PBX that will affect the Octel integration. Items not addressed here do not require modification for integration with the Octel 250 and should remain as-is on the PBX.

Configuring the PBX M-1 to integrate

o	Before you begin programming, it is recommended that a hard copy 	of the customer data block be printed in overlay 21 to verify the 	existing programming. If necessary, change the following parameters 	in overlay 	15 to the underlined values. Default values are in 	parentheses.

REQ		CHG

TYPE	CDB			Customer Data Block
	CUST	0-31			Customer Number
	OPT		MCI, (MCX)		Message Center is Included 							(excluded);  should be MCI
	MDID	YES, (NO)		No-answer DID calls are (are not)
						routed to the Message Center 							(Customer must choose if DID 							calls should route to the message 						center)
	NDID	YES, (NO)		No-answer non-DID calls are 
						(are not) routed to the Message
						Center (Customer must choose if 						NON-DID calls should route to 							the message center)
	MWFB	YES, (NO)		DID calls encountering busy are
						(are not) routed to Message 
						Center (Customer must choose if 						DID Busy calls should route to 							the message center)
	EEST	NO			Enhanced End-to-End Signaling
						tone (NO = OFF, YES = ON)
	LDTT	3 seconds min.		Line Disconnect Tone Timer - For
						Release 17 PBXs and above

o	It is recommended that a hard copy of the terminal number block 	(TNB) be run for M-1 sets and for 2500 sets to show existing 	programming. Use overlay 20 to retrieve this information.

## 5.1  Programming the Single-Line Ports

These ports are used for the voice path.

Programming the single line ports

o	Using overlay 10, program a unique directory number (DN) for each 	FLT port. Program the ports as follows:

REQ		NEW

TYPE	500			Single line interface data block
TN		lll S CC U		Terminal number: loop
CUST	0-31 or 0-99		Customer number
DN		XXXX			Directory number (see note)
HUNT	XXXX			Hunt to next key appearance
CLS		XFA			Transfer allowed
		DTN			Digit tone
		HTA			Hunting allowed
		LDTA			Line disconnect tone allowed 
					(Release 17 and above)

NOTE: The DN for each analog port must also be programmed on either channel A or B of the DMID as a single-call non ringing (SCN) key. This programming is covered on page 3 of this note under programming the M-1 TNs.

o	If desired, program the subscribers sets to forward to personal 	greeting under ring-no-answer, busy and/or all-calls conditions. In 	addition, assign message waiting to any station requiring message 	waiting notification. Program these changes in overlay 10 for 2500 	sets, and in overlay 11 for M-1 sets.

## 5.2  Configuring the M-1 Digital Terminal Number(s)

Define the stations that will connect the MID to the Octel 250. The requirements are as follows:

Configuring the M-1 digital terminal number(s)

For each 12-port DMID:
- One TN is configured as channel A of the DMID
- One TN is configured as channel C (for message waiting)

For each 24-port DMID:
- One TN is configured as channel A of the DMID
- One TN is configured as channel B of the DMID
- One TN is configured as channel C (for message waiting)

This section includes programming for the M-1 TNs associated with channel A and channel B (for 24-port DMIDs only). The programming for channel C (required for each DMID regardless of size) is found in Section 5.3 of this note.

o	Using overlay 11, program one M-1 TN that will be channel A of the 	DMID. Assign directory numbers (DNs) for keys 1-13 (excluding key 	7). Program as many keys as there are corresponding Octel Overture 	250 ports. When defining the keys, note that key 1 will correspond to 	FLT- port 1A, key 2 will correspond to FLT port 1B, etc., up to port 	2D with a 12 port MID.

Programming channel A of the DMID

NOTES:

•	Keys 07 and 14 are 'fixed' keys and cannot be programmed as a single-call, non-ringing (SCN) key. They should be skipped.

•	Each DN appearance assigned on keys 1-13 of a DMID channel must be a unique number that corresponds to a port on the FLT board. This is known as a multiple appearance.

•	The DN assigned to key 0 must be a phantom number. This key is used as the primary directory number (PDN) used to find the location of the programmed TN.

Channel A

REQ		NEW
TYPE	2616			Digital set data block
TN		lll S CC U		Terminal number: loop (0-159) 
					shelf (0-1) card (1-10) unit (0-7)
CDEN	SD, (DD), 4D		Density of this card is
		D8			single, (double), quad
CUST	0-31			Customer number
DES		OCTEL1		Designation for telephone
CLS		WTD			Warning tone denied
		ADD			Automatic digit display				CNDA*		Calling Name Display Allowed			DNDA*		Dialed Number Display Allowed

KEY		00 SCN XXXX		Phantom Number (not used) set 
					must have a PDN (Primary 
					Directory Number) 
		01 SCN XXXX		Single call non-ringing, (pilot 
					access #) DN, each station must 
					have a unique DN, and 
					correspond to a port on the TIC. 
					(i.e., port 1A)
		02 SCN XXXX		Single call non-ringing (port 1B)
		03 SCN XXXX		Single call non-ringing (port 1C)
		04 SCN XXXX		Single call non-ringing (port 1D)
		05 SCN XXXX		Single call non-ringing (port 1E)
		06 SCN XXXX		Single call non-ringing (port 1F)
		07 See Note
		08 SCN XXXX		Single call non-ringing (port 1G)
		09 SCN XXXX		Single call non-ringing (port 1H)
		10 SCN XXXX		Single call non-ringing (port 2A )
		11 SCN XXXX		Single call non-ringing (port 2B)
		12 SCN XXXX		Single call non-ringing (port 2C)
		13 SCN XXXX		Single call non-ringing (port 2D)
		14 See Note
		15 DSP 		Display Key

*  Required for support of Multiple Personal Greetings feature.

NOTE: Refer to section 6.1 of this note when assigning single call non-ringing (SCN) keys for multiple DMIDs.

o	If programming a 24-port DMID, program channel B of the DMID 	for Octel ports 2E-3H. Use overlay 11 to assign multiple appearance 	DNs to the appropriate keys on the M-1 digital terminal number as 	follows:

Programming channel B, for 24-port DMIDs only

Channel B

REQ		NEW

TYPE	2616			Digital Set Data Block
TN		lll S CC U		Terminal number: loop (0-						159) 
					shelf (0-1) card (1-10) unit 						(0-7)
CDEN	SD, (DD), 4D		Density of this card is
					single, (double), quad
CUST	0-31			Customer Number
DES		OCTEL2		Designation for telephone
CLS		WTD			Warning Tone Denied
		ADD			Automatic Digit Display			CNDA*		Calling Name Display Allowed			DNDA*		Dialed Number Display Allowed

KEY	00 SCN XXXX		Phantom Number (not 							used), set
					must have a PDN 							(Primary Directory Number)
		01 SCN XXXX		Single call non-ringing, DN, each 
					station must have a unique 						DN, and correspond to a port 						on the FLT-8 (i.e., port 2E)
		02 SCN XXXX 	Single call non-ringing 							(port 2F)
		03 SCN XXXX 	Single call non-ringing 							(port 2G)
		04 SCN XXXX 	Single call non-ringing 							(port 2H)
		05 SCN XXXX 	Single call non-ringing 							(port 3A)
		06 SCN XXXX 	Single call non-ringing 							(port 3B)
		07 See Note 
		08 SCN XXXX 	Single call non-ringing (port 3C)
		09 SCN XXXX 	Single call non-ringing (port 3D)
		10 SCN XXXX 	Single call non-ringing (port 3E)
		11 SCN XXXX		Single call non-ringing (port 3F)
		12 SCN XXXX 	Single call non-ringing (port 3G)
		13 SCN XXXX 	Single call non-ringing (port 3H)
		14 See Note
		15 DSP 		Display

*  Required for support of Multiple Personal Greetings feature.

NOTE:  Keys 07 and 14 are 'fixed' and cannot be programmed with an SCN. They should be skipped.

## 5.3  Programming the Message Center

For both 12-port and 24-port DMIDs, channel C is programmed as the message center that activates and deactivates message-waiting notification for subscribers’ telephones. The class of service should have High Priority assigned to increase the speed at which message waiting is set.

CAUTION:  If programming instructions are not followed; corruption of the DMID may occur and potentially requiring replacement of the DMID.

o	Using overlay 11, program channel C on the DMID as the message 	center, in the following order:

For both 12-port and 24-port DMIDs, channel C is programmed as the mesage center

REQ		ADD			ADD a set

TYPE	2008			Digital line interface
TN		lll S CC U		Terminal number: loop (0-159)
					shelf (0-1) card (1-10) unit (0-7)
CDEN	SD, (DD) 4D		Density of this card is single, 
					(double), quad
CUST	0-31 or 0-99		Customer number
DES		OCTEL3		Designation for Octel telephone
CLS		WTD			Warning tone denied
		ADD			Allow digit display				HPR			High Priority
KEY	00     SCN XXXX		Phantom number (not used), set 
					must have a PDN (primary 
					directory number)
		01 MIK			Message indication key
		02 MCK		Message cancellation Key

A phantom number should be programmed on key 0. It is used to define a PDN associated with the location of the TN.

Run a hard copy of the terminal number block (TNB) for the M-1 sets (channels A, B, and C) to reflect and verify the additions and changes just made. Use overlay 20.

## 5.4  PBX PROGRAMMING FOR multiple personal 	greetings

The following programming is required on the PBX to support the multiple personal greetings feature.  The programming necessary on the Octel system is specified under Menu 7 in section 6.0. To support multiple personal greetings, the DMID 9402 EPROM is required.

The following programming is required on the PBX to support the multiple personal greetings feature

o	For the load 11 programming for the digital TNs associated with the 	DMID, assign the following class-of-service features.  Note: This 	programming was included in section 5.2, Configuring the M-1 	Digital Terminal Number(s).

CNDA		Calling Name Display Allowed
		DNDA		Dialed Number Display Allowed

o	For personal terminal numbers (of  both subscribers and non-	subscribers), assign the following class-of-service features in load 10 	(for analog stations) and load 11 (for digital stations):

CNDA		Calling Name Display Allowed
		DNDA		Dialed Number Display Allowed

o	In load 95, Calling Party Name Display, set RESN to YES. Then 	assign the following values to the specified parameters:

CFWD	set to CFWD
		CFNA 	set to RNA
		HUNT	set to BUSY
		PKUP	set to PKUP
		XFER	leave at default
		AAA		leave at default

## 5.5  Optional Programming FOR ALL-PORTS-BUSY 	CONDITIONS

This is optional and is not required for all installations. For all ports-busy conditions, the PBX returns busy tone to the caller. If ringback tone is preferred rather than busy tone when all ports are busy, the C channel can be programmed as an ACD agent to provide queuing and ringing.

Optional programming for all-ports-busy conditions

The operation of this optional programming is as follows:

•	Calls that overflow to the channel C ring three times before being answered. Once answered, calls are immediately transferred back to the hunt group.

•	Forwarded calls that overflow to the C channel lose called party ID during the transfer back to the Octel 250 pilot number. Therefore, forwarded calls that overflow do not receive personal greeting.

•	Consultation hold calls cannot be transferred back into the hunt group.

•	If multiple DMIDs are required, the customer has agent ID login, and the overflow programming is desired, contact your nearest Octel Technical Analysis Center (TAC).

The programming steps are as follows:

o	An ACD queue must be created in overlay 23.

o	If the customer has agent ID login, '9999' must be reserved as the 	login ID for channel C.

o	Using overlay 11, program channel C on the DMID as the Message 	Center and an ACD agent, in the following order:

REQ		NEW		ADD a new set

TYPE	2008		2008
		TN		lll S CC U	Terminal number: loop (0-159)
						shelf (0-1) card (1-10) unit (0-7)
		CDEN	SD, (DD) 4D	Density of this card is single, 							(double), quad
		CUST	0-31 or 0-99	Customer Number
		DES		OCTEL3	Designation for Octel telephone
		CLS		WTD		Warning Tone Denied
				ADD		Allow Digit Display
		KEY	00 	ACD XXXX 	ACD Agent
			01 	MIK		Message Indication Key
			02 	MCK		Message Cancellation Key
			03 	TRN		Transfer Key
			04 	MSB		Make Set Busy

o	The last member in the hunt group must hunt to the ACD queue.

## 5.6  ACD Programming to Extend Hunt Group 	Capacity

M-1 option 11, 21, 51, or 61 installations with over 18 Octel integrated ports and M-1 option 71 or 81 installations with over 30 integrated Octel ports require additional ACD programming. This programming extends the capacity of the hunt group to accommodate the Octel 250 ports.

ACD programming to extend hunt group capacity

One ACD agent is needed for each hunt group in excess of 18 or 30 members, whichever is applicable.

o	In Overlay 23 program the night-call forward feature. The PBX 	technician should create an ACD agent as the last member of the first 	hunt group. The ACD agent should then be night-call forwarded to 	the next hunt group.

The last member in the hunt group must hunt to the ACD queue.

o	To view current programming in Overlay 23, at the initial REQ type 	PRT based on the application.

Assign the ACD DN (Directory Number), and night-call-forward target.

REQ		NEW		To add an ACD data block TYPE

TYPE	ACD		Automatic Call Distribution 
				data block 
CUST	0-31		Customer Number
ACDN	ACD		directory number (ACD-DN)
MWC	YES		Message Waiting Center
MAXP	1		Maximum Number of positions 
NCFW	XXXX		Night call forward DN for ACD
				(recommended this DN to be the
				operator or system manager).

## 5.7  Verify M-1 TNs Are Active

o	To ensure that the M-1 TNs are active, use overlay 32:

Verify that the M-1 TNS are active

: (Indicates ready to accept command)

STAT	XXXX		Where XXXX is the TN of
				the M-1 programmed as an
				Octel port

The status displayed will be either IDLE or DSBL.  IDLE status is required. If DSBL is displayed, enable them with the following in overlay 32:

ENLU	XXXX		Where XXXX is the TN of 
				the disabled TN

## 6.0  Configuring the Octel 250

¨	Menu 1.1  -  System Parameters

Configuring the Octel 250 system management menus

•	Company Phone Number : (see section 8.6)

•	VPMOD Phone Number:   Enter the Pilot Number of the Octel 250 (do not enter a prefix or an area code).

•	Type of PBX:  I - PBX DMID NT Meridian 1

§	Sender ID Used for Telephone Answering Msgs: 2 - Calling Party, if 							    Known

¨	Menu 4.1  -  Port Assignments

•	Ext. Field: In the extension field/phone number column, use the format 'LL AA' defined as follows:
LL is the fixed two digit link number , 01 to 03, see table.
AA is the variable length field (1 or 2 digits) for the bridged appearances, numbered 1 to 24 for each DMID, in order. Use leading zeroes for the link number (LL) only.

Table 1 shows an example of Menu 4.1 entries:

•	Line type: 79

•	M column: N for all ports

•	O column:  (see note)

•	N column:  (see note)

•	P column:  (see note)

NOTE:  These options are customer specific according to Outcalling, Networking, and Paging needs.

¨	Menu 4.3  -  RS-232 Message Waiting

•	Special RS-232 Message Waiting: Y

NOTE: All message waiting commands are sent over RS232 link #1 unless specified otherwise in individual mailbox profiles. For your installation, assign roughly the same number of subscribers to each RS232 link for message waiting using the 'Int. Link Number' parameter in the individual mailbox profiles. See the menu 8 and 9.1 programming step.

¨	Menu 6.1  -  Transfer Dialing Sequence

•	Flash Time: 500 ms

•	Pause Time: 1000 ms

§	Block Automated Attendant Transfer:    ,    ,    ,    .

•	Dialing Sequence to Transfer a Call: FPN

•	Dialing Sequence to Reconnect with a Call -
					Ring/No Answer:  F
						Busy:  F

¨	Menu 6.3  -  Define CPU Serial Channels

For each integration serial channel to be used, assign channel type '3 - 	Direct Integration'.  When 'Direct Integration' is chosen for a serial 	channel configuration, you will need to program menu 6.5.1 for each 	link, also.

NOTE: The Octel 250 must be rebooted to apply any changes to serial channel parameters.

¨	Menu 6.5  -  Integration Link Management:

§	Link Number: (1-3)

Link Name:
Type of Switch to which the System is Integrated: I - PBX DMID NT Meridian 1
Baud Rate: 3 - 9600 Baud
Number of Data Bits:  1 - 8 Data Bits
Number of Stop Bits:  0 - 1 Stop Bits
Parity (None/Odd/Even): 1 - Odd Parity
XON/XOFF: 1 - Ignore XON/XOFF
Carrier Detect: 1 - Carrier Detect Not Used
Extension to Check SMDI Message Waiting:  (leave blank)
Max. Msg. Waiting Operations per second: 1
Switch Number to which this Link is Associated: 1  
SMDI link down alarm threshold counter:  1

NOTE: After any changes are made in Menu 6.5.2, reset the affected integration link(s) using Menu 6.5.5.

¨	Menu 8/9.1 - Subscriber Mailbox Profile

Int. Link Number:  All message waiting commands are sent over RS232 link #1 unless specified otherwise in individual mailbox profiles. For your installation, assign roughly the same number of subscribers to each RS232 link for message waiting using this 'Int. Link Number' parameter in the individual mailbox profiles.  If not, delays in message waiting might occur.

## 6.1  Configuring Multiple DMIDs

On Octel 250s with greater than 24 integrated ports, additional DMIDs are required. Each additional 12-port DMID requires one M-1 digital terminal number for the integration and one terminal number for message waiting. Each additional 24-port DMID requires three M-1 digital terminal numbers, two for the integration and one terminal number for message waiting. Repeat the DMID programming instructions previously described for the additional DMIDs. Care should be taken in the following areas:

Configuring multiple DMIDS

•	One hunt group is used when multiple DMIDs are required. All 2616 line appearances must be in the same hunt group. The members of the group should be added in this specific order: from the first channel of the first DMID (DMID #1, channel A), then the first channel of the second DMID (DMID #2, channel A), then the first channel of the third DMID (DMID #3, channel A), then the second channel of the first DMID (DMID #1, channel B), etc. Channels should alternate between sets to provide load balancing between DMIDs.

•	The LTN is determined by the link number (01-03) and the port number assigned to the key that is associated to the 2616 digital station.

Example of multiple DMIDs:

Hunt Group	DMID		Channel 		Menu 4 (Ext./Phone Number)
	Member 5001	DMID#1	Channel A	011
	Member 5002	DMID#2	Channel A	021
	Member 5003	DMID#3	Channel A	031
	Member 5004	DMID#1	Channel B	0113
	Member 5005	DMID#2	Channel B	0213
	Member 5006	DMID#3	Channel B	0313
	Member 5007	DMID#1	Channel A	012
	Member 5008	DMID#2	Channel A	022
	Member 5009	DMID#3	Channel A	033
	Member 5010	DMID#1	Channel B	0114

## 6.2  Converting from an aMID to a DMID

When converting from an AMID to a DMID, use the programming described section 6.0, with the following changes:

Steps to convert from an AMID to a DMID

¨	Menu 1.1  -  System Parameters

•	VPMOD Phone Number:   Enter the Pilot Number of the Octel 250 (do not enter a prefix or an area code).

•	Type of PBX:  I - PBX DMID NT Meridian 1

¨	Menu 4.1  -  Port Assignments

•	Line Type: 79 (Change from line type 115 to line type 79)

Delete the ACD number and all agents associated with the AMID. Use the same pilot number as the original hunt number.

## 7.0  Installing the DMID

See Figure 1 for the physical connections to the DMID.

Installing the DMID

Refer to the installation instructions, p/n 181-1082-000, for detailed instructions on installing a DMID.

The 6-wire telephone lines plug into A, B, and C from the PBX to the DMID. If installing a 12-port DMID, use channel A and C only. If installing a 24-port DMID, use A, B, and C channels.

o	With the cables that shipped with the DMID, plug the RJ45 end into 	the DMID and the DB9 end into the CF board RS232 port.

o	Connect power by plugging the power-supply cord into a power 	outlet.

## 7.1  DMID DIP SWITCH SETTINGS

o	The DIP switch settings for an integrated 12-port DMID should be set 	as follows:

Setting DMID dip switches

o	The DIP switch settings for an integrated 24-port DMID should be:

## 7.2  Connecting the Analog Voice ports

Each FLT supports eight analog voice ports. Each FLT connects to the switch via a single 25 pair male amphenol connector cable. The first port uses the first pair of the cable, every other pair is used by the remaining ports.

Connecting the analog channels

¨	Physically connect the lines used for the voice path between the 	switch and the Octel 250. Each FLT provides  the amphenol 	connector on the front edge of the board.  See Figure 2.

## 7.3  CONNECTING THE dmid TO THE Octel 250

The CF board supports up to 3 integration links channels. See Figure 3.

Connecting the DMID to the Octel 250

o	The DB-9 connector should be fastened to the appropriate 	connector(s) on the CF board.

## 7.4  Powering the DMID

o	Insert the power cord into the power input receptacle and into a 110 	Vac wall outlet. The green status LED indicator should flash for 	approximately 5 minutes. After all the green LED indicators are ON 	steady, the DMID is initialized and functional. The green power LED 	indicator will remain activated as long as there is acceptable power to 	the DMID.

Powering the DMID

## 7.5  Testing the Installation

o	Create two mailboxes associated with two test extensions. Record a 	name and personal greeting for each mailbox.

Testing the installations when completed

o	Make sure that these extensions have been forwarded under busy and 	ring-no-answer conditions to the pilot number on the Octel Overture  	250 platform.

o	Using one test extension, call the other test extension. Verify that a 	personal greeting plays.

o	Leave a message. Verify that the message-waiting indicator turns on.

¨	Verify the Multiple Greetings feature for both busy and no answer 	conditions.

¨	Verify the Call Sender feature for both busy and no answer 	conditions.

o	Verify that the return-to-operator feature works properly.

o	Call the Octel 250 directly from a test extension. Verify that 	the recorded name is played immediately and that the system prompts 	for a password.

o	Review the message in the mailbox. Verify that the reply to the 	internal telephone-answering feature functions properly.

## 8.0  CONSIDERATIONS

8.1	The M-1 Option 11 PBX does not support reconnecting with a transferred call until Release 18. As a result, supervised transfers are not supported on the Option 11 at Release 17 and below and unsupervised transfers must be used.

Important notes regarding this integration

8.2	DMID integration uses hunting and does not require ACD unless the number of ports exceeds the number of hunting steps (18 steps are supported on the Options 21, 51, and 61, and 30 steps on the 71 and 81). If the number of ports exceeds the number of hunting steps, ACD is required to expand the hunting capability.

8.3	Verified and supported software versions are 15.55 to 22.xx. For questions regarding other versions, contact Sales Engineering.

8.4	Secretarial filtering allows a caller to forward his/her telephone to a secretary, permitting the designated secretary to ring the station of the forwarded party even though the telephone is in an all-calls-forward mode. If a subscriber has his/her station all-calls forwarded to the Octel 250, the first port of the voice-processing module hunt group acts as the secretarial position. Therefore, if the first port of the Octel 250 hunt group performs a transfer to a station that is all-calls forwarded to the Octel 250, the station will ring instead of forwarding the call.

8.5	Meridian M-1 PBXs require Release 17 or greater to support a positive disconnect for 2500-single line sets - with the exception of Option 11 PBXs in Canada which require Release 18. Octel strongly recommends that the PBX be at a Release that supports disconnect signaling. If disconnect signaling is not supported, the Octel 250 must rely on silence detection to disconnect. If callers and subscribers do not exit the Octel 250 properly, ports might take longer to disconnect causing extra port traffic. It is recommended that subscribers be trained to exit the Octel 250 completely when finished (by “starring” out) and that limited or extended caller prompting is set (via class of service) to prompt outside callers to also exit properly.

8.6	ISDN/SS7 trunks forward the originating telephone number of all outside calls to the Octel 250. If the telephone number matches an existing mailbox number, external calls might appear to have been sent by internal subscribers, allowing message recipients to automatically reply to an incorrect mailboxes. This may also happen if mailbox lengths are fewer than seven digits. With Release 1.55, the “Company Phone Number” field in Menu 1.1 will resolve this situation. Refer to Service Note 93-235 for programming details.

8.7	Northern Telecom supports two types of ACD overflow. The "Overflow" option sends calls in queue to a designated back-up after a predetermined time limit. The "Interflow" feature sends calls in queue to a back-up when the queue size reaches a predetermined maximum.

For DMID integrations only, calls that "Overflow" from an ACD queue to the Octel 250 lose called party ID. (This is not true for the MID which, unlike the DMID, requires the Octel ports to be an ACD group.) However, calls that "Interflow" from an ACD queue to the Octel Overture system retain called party ID. Check with Northern Telecom regarding the requirements and limitations of the ACD "Interflow" feature.

  - Customers may experience a problem with Nortel Release 25.10, Where the PBX does not provide the correct display information needed to integrate properly. Customers must request that Nortel install patch MPLR 12969 in order to resolve this problem.

## 9.0  Addendum A - Northern Telecom’s Network Message Services

The Northern Telecom Network Message Services - Message Center feature is designed to allow a centralized message center to serve multiple Northern Telecom PBXs. With this feature an Octel system, acting as a message center, can provide integration for multiple Meridian SL-1 or Meridian M-1 PBXs connected with an ISDN or T-1/D-channel network. Remote subscribers receive all integration features. The Digital Meridian Integration Device (DMID) is required; IVMS is not supported by Network Message Services - Message Center.

The Northern Telecom Network Message Services feature requires one of the following Northern Telecom networking capabilities: ISDN Primary Rate Access (PRA) or ISDN Signaling Link (ISL). Both require a signaling path, called the D-channel. PRA networks utilize the D channel of the PRA to carry signaling information. ISL networks utilize a dedicated data line as a “D channel”, usually requiring modems.

To support an ISL network for integration, the D-channel must operate at 9600 bps or faster. Slower speeds can cause message waiting notification to become unreliable. There is no restriction for PRA networks, which support a D-channel operating at 56 kbps.

Below is a summary list of the PBX requirements. This list is not intended to be all inclusive; it is meant as a guideline only. Some options listed have hardware and software prerequisites that are not listed. It remains the responsibility of the customer and PBX vendor to identify all necessary hardware and software components and to correctly configure the Northern Telecom Network Message Services - Message Center.

## NORTHERN TELECOM PBX REQUIREMENTS:

•	Integration Requirements for Local PBX - refer to appropriate Configuration Note

•	X11 Release 16 Software

•	Network Message Services (option 175)

•	Advanced ISDN Network Features (option 148)

•	ISDN Signaling (option 145)

•	ISDN Primary Rate Access (PRA) (option 146) or ISDN Signaling Link1 (ISL) (option 147)

•	Message Center (option 46)

•	End-to-End Signaling (option 10)

•	Coordinated Dial Plan (CDP) (option 59) or Network Automatic Route Select (NARS)(option 58)

¹ With ISL networking, the D-channel must operate at a minimum of 9600 bps.

©2001 AVAYA Inc. All rights reserved. All trademarks identified by the ®, SM and TM are registered trademarks, servicemarks or trademarks respectively. All other trademarks are properties of their respective owners. The above information is based on knowledge available at the time of publication and is subject to change without notice. Printed in U.S.A.

AVAYA Inc.

1001 Murphy Ranch Road

Milpitas, CA 95035-7912

(408) 321-2000

http://www.avaya.com