Configuration Note 5402 - Ver. J (05/02)

Octel 250

Message Server

Northern Telecom Meridian M-1*(FLT-N)

Linear Hunt

## 1.0	method of integration

The digital set emulation integration on the Nortel Meridian One, the FLT-N within the Octel system appears as one or more digital 2616 type telephones to the PBX.  When a call is presented to a hunt group consisting of FLT-N ports, the display information is read and the call is then answered with the appropriate greeting. Message Waiting is set using the message- indication and message-cancellation keys on the 2616 and cleared over FLT-N Ports.

The FLT-N emulates M2616 digital telephone sets

2.0	octel ordering information

Octel 250 requirements

- FLT-N(s) - Each FLT-N supports 8 Ports

- Set Emulation Software feature (Feature Bit 33)

- RS232 Integration feature (Feature bit 34)

- In-band Integration feature (Feature bit 35)

## 3.0	PBX HARDWARE requirements

§	NT8D02 Integrated Services Digital Line (ISDL) ports:
- Revision AB or higher

§	25-pair Amphenol cables (male end), one per FLT-N

## 3.1	PBX SOFTWARE REQUIREMENTS

•	A minimum of Generic X.11, Release 15.00 with the following:

PBX software requirements

•	BACD, Basic Automatic Call Distribution, Option 40 see section 8.2

•	MSB, Make Set Busy, Option 17

•	ACDA, Automatic Call Distribution, Option 45

•	MWC, Message Waiting Center, Option 46

•	EES, Enhanced End-to-End Signaling, Option 10

•	DDSP, Digit Display, Option 19

## 4.0	SUPPORTED FEATURES

•	Station forward to personal greeting
- all calls
- RNA									- Busy

Supported integration features

•	System forward to personal greeting
- busy
- RNA

•	Message-waiting Indicator
- lights
- audible message waiting (stutter dial tone)

•	Automated attendant						   - Supervised							   - Unsupervised

•	Outcalling

•	Multiple return-to-operator

•	Direct call

•	Personal greeting of original-called party on double-call forward

•	Reply to message left by subscriber in internal telephone-answering mode

•	Call sender

•	Multiple Personal Greetings					      - RNA								           - Busy

## Configuring the M-1 PBX

SPECIAL NOTE: The following represents changes to the M-1 PBX that will affect the Octel integration. Items not addressed here do not require modification for integration with the Octel 250 and should remain as-is on the PBX.

Configuring the M-1 PBX

o	It is recommended that a hard copy of the Customer Data Block be 	printed in overlay 21 to verify the existing programming. If 	necessary, 	change the following parameters in overlay 15 to the 	underlined values. 	Default values are listed in parentheses.

REQ		CHG

TYPE	CDB			Customer Data Block

CUST	0-31			Customer Number

OPT		MCI, (MCX)		Message Center is
					Included (excluded)

MDID	YES, (NO)		No-answer DID calls
					are (are not) routed 
					to the Message Center

NDID	YES, (NO)		No-answer non-DID
					calls are (are not) 
					routed to the Message
					Center

MWFB	YES, (NO)		DID calls 
					encountering busy are
					(are not) routed to 
					Message Center

EEST	NO			Enhanced end-to-end signaling 
					(OFF)

5.1	Configuring the 2616 Terminal NUMBER (s) in a hunt group

Programming the digital ports in a hunt group

One 2616 TN is configured for each FLT-N port with Linear Hunt Configuration.

The DN assigned to key 0 is the primary directory number (PDN) used as part of the hunt group.

2616 Set

REQ	NEW

TYPE	2616			Digital set data block

TN		lll S CC U		Terminal number: loop (0-159) 
					shelf (0-1) card (1-10) unit (0-7)

CDEN	SD, (DD), 4D		Density of this card is
					single, (double), quad and is dependent on 					the type of PBX card is being used

CUST	0-31			Customer number

DES		OCTEL1		Designation for telephone

CLS		FBD			Forward Busy Denied
		HTA			Hunting Allowed
		MWD		Message Waiting Denied
		CNDA		Calling Name Display Allowed				DNDA		Dialed Number Display Allowed			CPTD		Forced Camp-on to another set Denied

KEY	00 SCR XXXX	PDN (Primary 
					Directory Number Port 1) 
		06 MSB		Make Set Busy						13 MIK		Message Indication Key
		14 MCK		Message Cancellation Key				15 TRN		Transfer Key

Program one 2616 station for each FLT-N port

## NOTE:

§	Keys not mentioned should be configured as Blank keys.

## 5.4	Configuring the Subscribers telephone

2616 Set

REQ	CHG

TYPE	2616			Digital set data block

TN		lll S CC U		Terminal number: loop (0-159) 
					shelf (0-1) card (1-10) unit (0-7)

CDEN	SD, (DD), 4D		Density of this card is
		D8			single, (double), quad

CUST	0-31			Customer number

DES		XXXXX		Designation for telephone

FDN 	XXXX		Forward Directory Number (Enter the 						Pilot number of the Octel server)

CLS		FBA			Forward Busy Allowed					FNA			Forward Ring-No-Answer Allowed
		HTA			Hunting Allowed
		MWA		Message Waiting Allowed
		CNDA		Calling Name Display Allowed				DNDA		Dialed Number Display Allowed

HUNT	XXXX		Hunt DN of next station (Enter the Pilot 					number of the Octel server)

KEY	00 SCR XXXX	PDN (Primary 
		01 SCN XXXX	This number must be unique for each 						2616 set 						xx MWK XXXX	Message Waiting Key can be 							programmed on any other available key.

## 5.5	PBX PROGRAMMING FOR multiple personal 	greetings

The following programming is required on the PBX to support the multiple personal greetings feature.  The programming necessary on the Octel 250 server is specified under Menu 7 in section 6.0.

The following programming is required on the PBX to support the multiple personal greetings feature

o	In Overlay 95, Calling Party Name Display set RESN to YES. Then 	assign the following values to the specified parameters:

REQ		CHG								TYPE	CPND								CUST									CNFG									MXLN									STAL									DFLN									DES										RESN	YES								CFWD	CFWD								CFNA 	RNA								HUNT	BUSY								PKUP	PKUP								XFER	leave at default							AAA	leave at default

## 5.6	Verifying that M-1 TNs are Active

o	To ensure the M-1 TNs are active, go to overlay 32.

Verifying that M-1 TNS are active

•	(Indicates ready to accept command)

STAT	XXXX		Where XXXX is the TN of
					the M-1 programmed as an
					Octel port

The status displayed will IDLE or DSBL

IDLE status is required. If DSBL is displayed, enable them with the following using overlay 32:

ENLU		XXXX		Where XXXX is the TN of 
					the disabled TN

## 6.0	Configuring the Octel 250

o	Menu 1.1 - System Parameters

Configuring the Octel 250 for up to 24 ports

§	Type of Switch connected to: I - PBX DMID NT Meridian 1

§	VPMOD telephone number: This is the pilot number of the Hunt Group - Not the full DDI, only the extension number, e.g.2250.

§	Number of Digits in Extension used for Outcalling and ECP:

§	Number of Digits in Extension used for Message Waiting:

§	Sender ID Used for Telephone Answering Messages: Calling if known

o	Menu 4.1 - Port Configuration for LINEAR HUNT

§	Extension/Phone No.: This field is not used for Hunt Group Support

§	Incoming: Y

§	Line Type: 148

§	D:

§	M: Y (Enter “Yes” under the “M”.)

It is recommended to dedicate ports for Message Waiting, otherwise users could experience delays.

o	Menu 4.3  - RS-232 Message Waiting

§	Special RS-232 Message Waiting? N

NOTE:	 When moving from AMID/DMID method to the FLT-N integration, Special RS-232 Message Waiting must be set to No, otherwise message waiting will not function.

Note:  All other entries are customer application specific.

o	Menu 4.2 - RS-232 Message Waiting

§	Special RS-232 Message Waiting? N

o	Menu 8/9.1 - Subscriber Mailbox Profile

§	Subscribers Extension:

§	Message Waiting Allowed: Y

§	Int. Link Number: N/A

o	Menu 7 - Class of Service Profile:

For subscribers allowed multiple personal greetings, set their class of service parameter Personal Greeting to 2 - Multiple Greetings.  Be sure to remind them to record both greetings within their mailbox.

7.0	CONNECTING THE FLT-N

Connecting the FLTs

o	Each FLT supports eight digital voice ports. Each FLT connects to 	the switch via a single 25 pair male amphenol connector cable. The 	first 	port uses the first pair of the cable; the remaining ports use every other pair.

## 7.2	Troubleshooting - Verify M-1 TNs Are Active

o	To ensure the M-1 TNs are active, go to overlay 32.

Verifying the M-1 TNs are active

•	(Indicates ready to accept command)

STAT	XXXX		Where XXXX is the TN of
					the M-1 programmed as an
					Octel port

The status displayed will be IDLE or DSBL

IDLE status is the status desired. If DSBL is displayed, enable them 	with the following using overlay 32.

ENLU	XXXX		Where XXXX is the TN of 
					the disabled TN

## 7.3	Testing the Installation

o	Create two mailboxes associated with two test extensions. Record a 	name and personal greeting for each mailbox.

Testing the installation

o	Make sure these extensions have been forwarded under busy and no-	answer conditions to the Octel 250 pilot number.

o	Using one test extension, call the other test extension. You should 	hear the personal greeting.

o	Leave a message. Verify that the message-waiting indicator turns on.

o	Verify that return-to-operator works properly.

o	Call the Octel 250 from a test extension. You should 	immediately hear the recorded name and be asked to enter your 	password.

o	Review the message in the mailbox. Verify that you can 	automatically reply to internal telephone-answering messages.

## 8.0	CONSIDERATIONS

The Meridian Option 11 does not support supervised transfers below Release 18.  This is because the option 11 does not currently support reconnecting with a transferred call, Unsupervised transfers must be used.

Important notes regarding this integration

- Determine if ACD option is required. FLT-N integration installations that use hunting, does not require ACD unless the number of ports exceeds the number of hunting steps available on your PBX model. Eighteen steps are supported on the Options 11, 21, 51, or 61. Thirty steps are available on the Options 71 and 81. If the number of ports exceeds the number of hunting steps, ACD is required to expand the hunting capability.

- Verified versions of PBX software include 15.00 through 25.xx.

- Secretarial filtering allows a set (phone A, for example) to forward calls to a second set (phone B). If a user at phone B transfers such forwarded calls back to phone A, phone A will ring instead of re-forwarding the call.

- No more than 8 voice mail ports on the NT PBX cards are recommended. Octel highly recommends that the Octel voice ports be distributed among different port cards/shelves/loops on the M1 PBX. This reduces the possibility that a single card/shelve failure will affect a large number of Octel ports.  Depending on the PBX architecture, performance could also be an issue on some PBXs during high traffic, if a large number of calls are being processed on the same card.  In addition, heavy traffic could cause the PBX to reboot and or initialize.

- . The 2616 station configured for message waiting should be assigned on an NT card with low traffic. Otherwise, customers can experience excessive 7A (message waiting bad extension) errors, on valid extensions.

- Octel recommends a ratio of one dedicated message waiting port for every 24 integrated ports.  However some customer environments may require a higher ratio due to MWI traffic.

- When converting from DMID integration to the FLT-N integration, be sure that Menu 4 has “Special RS-232 Message Waiting” set to NO, before shutting down the system and installing the FLT-N.

- Automated Attendant/ECP applications transferring calls between NMS nodes may experience loss of integration.  When Trunk Route Optimization is enabled on NMS trunks, calls transferred to stations on remote switches that forward without first ringing the station do not pass called party ID to the FLT-N port. The workaround is to have customer’s use Trunk Tromboning that allows call ID to be passed throughout the entire call process.

- Avaya recommends the “Background Task 30” be disabled in the configuration data block (LD 17) of the M1.  Otherwise, problems with integration and message waiting can occur. This background task 30 can be performed during the midnight routine, however is not recommended as a background routine.

- F9 errors.  During the PBX midnight routine, when Overlay 30 runs, the voice server may log F9 errors (25/28) accompanied by XMI clock errors.  These errors are non-service affecting and are the result of a problem with the PBX clocking mechanism.  This problem has been resolved with Aria 3.0 software.

- Customers may experience a problem with Nortel Release 25.10, where the PBX does not provide the correct display information needed to integrate properly. Customers must request that Nortel install patch MPLR 12969 in order to resolve this problem.

## 9.0	Addendum A - Northern Telecom’s Network Message Services

The Northern Telecom Network Message Services - Message Center feature is designed to allow a centralized message center to serve multiple Northern Telecom PBXs. With this feature an Octel system, acting as a message center, can provide integration for multiple Meridian SL-1 or Meridian M-1 PBXs connected with an ISDN or T-1/D-channel network.

The Northern Telecom Network Message Services feature requires one of the following Northern Telecom networking capabilities: ISDN Primary Rate Access (PRA) or ISDN Signaling Link (ISL). Both require a signaling path, called the D-channel. PRA networks utilize the D channel of the PRA to carry signaling information. ISL networks utilize a dedicated data line as a “D channel”, usually requiring modems.

To support an ISL network for integration, the D-channel must operate at 9600 bps or faster. Slower speeds can cause message-waiting notification to become unreliable. There is no restriction for PRA networks, which support a D-channel operating at 56 kbps.

Below is a summary list of the PBX requirements. This list is not intended to be all-inclusive; it is meant as a guideline only. Some options listed have hardware and software prerequisites that are not listed. It remains the responsibility of the customer and PBX vendor to identify all necessary hardware and software components and to correctly configure the Northern Telecom Network Message Services - Message Center.

## NORTHERN TELECOM PBX REQUIREMENTS:

•	Integration Requirements for Local PBX - refer to appropriate Configuration Note

•	X11 Release 16 Software

•	Network Message Services (option 175)

•	Advanced ISDN Network Features (option 148)

•	ISDN Signaling (option 145)

•	ISDN Primary Rate Access (PRA) (option 146) or ISDN Signaling Link1 (ISL) (option 147)

•	Message Center (option 46)

•	End-to-End Signaling (option 10)

•	Coordinated Dial Plan (CDP) (option 59) or Network Automatic Route Select (NARS)(option 58)

¹ With ISL networking, the D-channel must operate at a minimum of 9600 bps.

©2002 AVAYA Inc. All rights reserved. All trademarks identified by the ®, SM and TM are registered trademarks, servicemarks or trademarks respectively. All other trademarks are properties of their respective owners. The above information is based on knowledge available at the time of publication and is subject to change without notice. Printed in U.S.A.

AVAYA

911 Murphy Ranch Road

Milpitas, CA 95035-7912

(408) 577-7000

http://www.avaya.com