    [image: image2.png]AVAYA




[image: image1.png]AVAYA




Communication Manager 3.x Software & Firmware Compatibility Matrix  

Version 2 – November 7th, 2005


Communication Manager 3.x Software & Firmware Compatibility Matrix  

Version 2 –November 7th, 2005

[image: image1.png]

Table of Contents

4Communication Manager 1.x Releases


4Communication Manager 2.x Releases


4General Software / Firmware Compatibility Guidelines


4Issues Specific To 3.x Releases


4Product Correction Notices for 3.x Releases


4New Installations for 3.x Releases


5Product Documentation


5Communication Manager 1.x to Communication Manager 3.0.1 Upgrades


6Communication Manager 2.x to Communication Manager 3.x Upgrades


7S8300A Media Server Support


7DEFINITY Server SI Support


7Service Packs for Communication Manager 3.x Releases


8Media Gateway, LSP, and Primary Server Compatibility Issues for Current Releases


8Telephone Software Dependencies


9Circuit Pack Firmware Compatibility


9IP Interface Circuit Pack Firmware Available on the Communication Manager 3.x Software CD


9The Latest Firmware for all Programmable TN Circuit Packs:


9TN2214CP:


9TN2224CP:


10TN2302AP:


10TN2312AP:


10TN2313AP:


10TN2464BP:


10TN2501AP:


10TN2602AP:


10TN464GP:


10TN771DP:


10TN799DP:


10Media Gateway Firmware Compatibility


11Gateway/Server/LSP Compatibility Matrix


11The Latest Firmware for G250, G350, and G700 Media Gateways and Media Modules:


12Telephone Software Compatibility


122400 Digital Telephones:


12All 46xx IP Telephones:


126400 Digital Telephones:


128400 Digital Telephones:


12IP Agent:


12IP Softphone:


12IP Softphone for Pocket PC:


12Softconsole:


13Service Pack Compatibility


13Service Packs for all releases:


13Communication Manager 1.3:


13Communication Manager 1.3.1:


13Communication Manager 1.3.2:


13Communication Manager 2.0:


13Communication Manager 2.0.1:


14Communication Manager 2.1:


14Communication Manager 2.1.1:


14Communication Manager 2.2:


14Communication Manager 2.2.1:


14Communication Manager 3.0:


14Communication Manager 3.0.1:


14E-Notifications:


14Integrated Management


14Fault and Performance Manager:


15MultiSite Administration:


15Network Management:


15Proxy Agent:


15Site Administration:


15Voice Announcement Manager:


15VoIP Monitoring Manager:


15Software/Firmware Compatibility Matrix


16S8100 MEDIA SERVER


16G150 MEDIA GATEWAY COMPATIBILITY


17G250 (MGP only), G350, & G700 MEDIA GATEWAY COMMON FIRMWARE


18G350 MEDIA GATEWAY SPECIFIC FIRMWARE


18S8300, S8500, & S8700 SERIES MEDIA SERVERS


20DEFINITY SERVER SI / CSI / R





Communication Manager 1.x Releases

Communication Manager 1.x releases are included in the Compatibility Matrix.  However, detailed information regarding compatibility of Communication Manager 1.x releases is no longer provided.

Communication Manager 2.x Releases

Communication Manager 2.x releases are included in the Compatibility Matrix.  However, detailed information regarding compatibility of Communication Manager 2.x releases is provided in an earlier document. This document can be accessed on support.avaya.com at the following link:

http://support.avaya.com/elmodocs2/comm_mgr/CM_2_x_sw_fw_compatibility_Matrix_v6.doc
General Software / Firmware Compatibility Guidelines

The G150, G250, G350 & G700 Media Gateways, media modules, embedded P330 Ethernet switching system, various circuit packs, and many endpoints are designed to allow remote firmware upgrades once they’ve been installed in the field.  Furthermore, the S8300, S8500, and S8700 series Media Servers allow Communication Manager software to be updated via downloadable service packs.

New versions of firmware for media gateways, media modules, circuit packs, and endpoints continue to be developed and become available on the support web pages on an ongoing basis.  New versions of firmware usually contain support for new functionality as well as fixes for known issues with previous firmware vintages.  Therefore, it is strongly recommended to update firmware as new vintages become available.  However, it is not required, especially if a system is experiencing no problems on the current version of firmware and new functionality is not desired. Furthermore, there are firmware compatibility issues on Media Gateways.  Refer to the Software/Firmware Compatibility Matrix for details on compatibility.

Whenever updating vintages of firmware, it is extremely important to review the readme file associated with the firmware vintage.  The readme files document any known caveats or issues with existing Communication Manager software loads.  While the general rule is that the latest firmware is always backwards compatible with older Communication Manager software releases, there are sometimes exceptions, and these exceptions, when known, are documented in the readme files.  There are known backward compatibility exceptions where Media Gateways and LSPs are concerned.  Refer to the Media Gateway, LSP, and Primary Server Compatibility or Software/Firmware Compatibility Matrix sections for additional details.

For the S8300, S8500, and S8700 series Media Servers the latest service pack must be applied. For additional information see the Service Pack Compatibility section contained in this document.

Issues Specific To 3.x Releases

Product Correction Notices for 3.x Releases

All service packs and new vintages of firmware are handled via Product Correction Notices (PCN). Product Correction Notices contain important information about the new software or firmware and provide automated e-mail notifications.

New Installations for 3.x Releases

When installing Communication Manager 3.0 and 3.0.1 on a new S8300B Media Server, the installation fails if the hard drive is loaded with Remaster Program (RP) 3-11 software and TFTP via the services port is the installation method used.  The version of RP software on the S8300 hard drive can be checked via the label affixed to the top of the hard drive.  A CD-ROM drive and the Communication Manager 3.0 or 3.0.1 software CD can be used to workaround this issue.

There is no software shipped on the hard disk drive for the S8300, S8500, and S8700 series Media Servers.  The hard drive must be formatted and populated with software by using the Communication Manager software CD. For the S8300, the hard drive can alternatively be populated with software by using TFTP and the services port on the Media Server.

Communication Manager 3.x releases do not install or run on the S8300A Media Server.  An S8300B Media Server is required to run Communication Manager 3.x releases.
Product Documentation

The latest up-to-date installation and upgrade product documentation can be found on support.avaya.com by doing the following:

1. Enter http://support.avaya.com in a web browser.
2. Select FIND DOCUMENTATION and DOWNLOADS by PRODUCT NAME
3. Select the appropriate product (e.g., S8700 and S8710 Media Servers) 
4. Select Installations, Migrations, Upgrades & Configurations
5. Select the desired release and document
Documentation for the Communication Manager 3.0.1 Connection Preserving Upgrades and Security of IP Telephone Registration/H.323 Signaling Channel features can be found in the CM 3.0.1 release notes Documentation Updates section. These documents can be obtained by doing the following:

1. Enter http://support.avaya.com in a web browser.
2. Select FIND DOCUMENTATION and DOWNLOADS by PRODUCT NAME
3. Select the appropriate product (e.g., S8700 and S8710 Media Servers) 
4. Select Release 3.0/3.0.1
5. Select Release Notes & Software Update Notes
6. Select Release Notes for Release 3.0.1 of Communication Manager
7. Select View HTML
8. Select Documentation Updates
9. Select the Updates document, then select the appropriate document link within the Updates document
Communication Manager 1.x to Communication Manager 3.0.1 Upgrades

CM 1.x (loads 110.4, 526.5, and 536.1) to 3.x high-level S8700 Media Servers upgrade steps:

Note: Any upgrade from 1.x to 3.x requires a new license file.

1. Use the appropriate CM 3.x Product Documentation on support.avaya.com.

CM 1.x (loads other than 110.4, 526.5, and 536.1) to 3.x high-level S8700 Media Server upgrade steps:

Note: Any upgrade from 1.x to 3.x requires a new license file.


1. Perform the usual pre-upgrade activities such as checking for alarms, observing the server for busyout conditions, etc.

2. Contact the RTS to acquire an erase_mbr pre-upgrade patch for the appropriate 1.x software load. Then, apply the pre-upgrade patch on the Media Server.

3. Backup the server using the Linux Migration Backup/Restore.

4. Put the CM 3.x software CD in the CD-ROM drive and issue the erase_mbr command from the bash. The erase_mbr command can be run by a craft login.

5. Reboot the server. The server should boot into the CM 3.x installation software on the CD-ROM. This can be verified by ensuring the installation screens are in color.

6. Perform the CM 3.x installation using the “Installing and Configuring the Avaya S8700 Series Media Server, Release 3.0” document. Specifically, refer to Chapter 3, “Installing Communication Manager Software” pages 37 – 41.

7. Apply the latest Communication Manager 3.x Service Pack on the Media Server. The latest CM 3.x Service Pack is available on support.avaya.com.

8. Restore the server from the backup.

9. If the load being upgraded from is earlier than 1.3.1 (load 531), reconfigure the server using the Maintenance Web Pages.

CM 1.x to 3.0.1 high-level S8300 Media Server upgrade steps:

Note: Any upgrade from 1.x to 3.0.1 requires a new license file. 

1. If the S8300 server is an S8300A this upgrade will require a migration to an S8300B Media Server. The S8300B can be distinguished from the S8300A because the S8300B is labeled “S8300B” on the faceplate and the color of the faceplate is gray.  To distinguish an S8300B server from an S8300A server remotely, a “free –b” command can be issued from the bash to determine the total memory available on the server.  The S8300B server will show total memory available as “528072704” and an S8300A will show less total memory available.  

2. Backup the server via Linux Migration Backup.

3. If the existing server is an S8300A, replace it with an S8300B. This is not a free upgrade. Perform a standard S8300A to S8300B Media Server migration without completing the restore step on the server. Use the “Installation and Upgrades for Avaya G700 Media Gateway and Avaya S8300 Media Server document. Specifically, refer to Chapter 5, “Upgrading an existing S8300A to R2.2” pages 209-252. Stop at the step just before the section header “Restoring Data”.

4. Apply the latest Communication Manager 3.0.1 Service Pack on the Media Server. The latest CM 3.0.1 Service Pack is available on support.avaya.com.

5. Restore the server from the backup.

6. Configure the server using the Maintenance Web Pages.

7. Complete the remainder of the upgrade steps. Refer to the “Installation and Upgrades for Avaya G700 Media Gateway and Avaya S8300 Media Server” document. Specifically, refer to Chapter 5, “Upgrading an existing S8300A to R2.2” pages 263-280.

Communication Manager 2.x to Communication Manager 3.x Upgrades

CM 2.x (load 219.0, 221.1, 411.7, 414.1, and 111.4) to 3.x high-level S8300, S8500, and S87x0 Media Server upgrade steps:

Note: Any upgrade from 2.x to 3.x requires a new license file. 

1. Use the appropriate Product Documentation on the documentation CD or on support.avaya.com.

CM 2.x (loads other than 219.0, 221.1, 411.7, 414.1, and 111.4) to 3.x high-level S87x0 Media Server upgrade steps:

Note: Any upgrade from 2.x to 3.x requires a new license file.  

1. Backup the server.

2. Place the CM 3.x software CD in the CD-ROM drive. Run the remaster command from the bash and reboot the server. Note: the remaster command is not available on CM releases earlier than 2.0.1.

3. The server will boot to the original installation software, which will be running a black-and-white character mode installation. The original installation software will load and boot into the CM 3.x installation software from the CD-ROM. This can be verified by ensuring the installation screens are in color after the second reboot.

4. Perform a standard 3.x installation using the “Installing and Configuring the Avaya S8700 Series Media Server, Release 3.0” document. Specifically, refer to Chapter 3, “Installing Communication Manager Software”.

5. Restore the server from the backup.

CM 2.x (loads other than 219.0, 221.1, 411.7, 414.1, and 111.4) to 3.0.1 high-level S8300 and S8500 Media Server upgrade steps:

Note: Any upgrade from 2.x to 3.0.1 requires a new license file.  

1. Backup the server.

2. Place the CM 3.x software CD in the CD-ROM drive. Run the remaster command from the bash and reboot the server. Note: the remaster command is not available on CM releases earlier than 2.0.1.

3. The server will boot to the original installation software, which will be running a black-and-white character mode installation. The original installation software will load and boot into the CM 3.x installation software from the CD-ROM. This can be verified by ensuring the installation screens are in color after the second reboot.

4. Perform the 3.x installation using the appropriate installation document for the media server .

5. Install the latest CM 3.0.1 Service Pack.

8. Restore the server from the backup.

9. Complete the remainder of the upgrade steps.

S8300A Media Server Support

Communication Manager 2.1 and later releases cannot be upgraded to or run on an S8300A Media Server.  An S8300B Media Server is required to run Communication Manager 2.1 and later releases.

DEFINITY Server SI Support

Communication Manager 3.0 and later releases are not supported on a DEFINITY Server SI.
Service Packs for Communication Manager 3.x Releases

There are two service packs for Communication Manager 3.0. One service pack is for SIP enabled configurations and the other is for non-SIP enabled configurations. The service pack for SIP enabled configurations is not call preserving when it is activated.

For Communication Manager 3.0.1 there is one service pack for both SIP enabled and non-SIP enabled configurations.

All service packs are issued with Product Correction Notices (PCN).  Product Correction Notices contain important information about the service pack and allow for automated notification via e-mail when new service packs become available.

The application of service packs is call preserving unless noted otherwise in the PCN.  This means that existing calls remain stable while the service pack is being applied.  Additionally, new calls can be initiated and signaling on existing calls is processed while the service pack is being applied.

Please refer to section Service Pack Compatibility for additional information on service packs.

Media Gateway, LSP, and Primary Server Compatibility Issues for Current Releases

G700 Media Gateway firmware vintages 24.xx.x (e.g., 24.17.0, 24.21.1, etc.) are not compatible with primary Media Servers and LSPs running Communication Manager releases earlier than 3.0. For Communication Manager 2.x releases, G700 Media Gateway firmware vintages 23.xx.x should be used.

The G350 Media Gateway is not supported in configurations with primary Media Servers and LSPs running Communication Manager releases earlier than 2.0.

The G250 Media Gateway is not supported in configurations with primary Media Servers and LSPs running Communication Manager releases earlier than 3.0.

The G150 Media Gateway is not supported in configurations with primary Media Servers and LSPs running Communication Manager releases earlier than 2.2.

G700 Media Gateway firmware v22.8.0 and earlier vintages are incompatible with S8300 Media Servers using IA770 and running Communication Manager 2.1 and later releases due to port assignments.  The Media Gateway firmware must be upgraded to the latest supported version for an S8300 Media Server using IA770 and running Communication Manager 2.1 or later software.

A maximum of 3 MM717 24-port DCP Media Modules can be installed in a single G700.  The number of MM717 Media Modules supported is limited by power draw.  Also, the ports on the MM717 are intended for in-building use only.  Phone lines connected to those ports are not to be routed out-of-building. Failure to comply with this restriction could cause harm to personnel or equipment.
There are currently 3 distinct Media Module 711 (MM711) firmware vintages for 3 distinct MM711 hardware vintage ranges.  These distinct firmware vintage ranges are incompatible with any other MM711 hardware vintages than the ones for which they were intended.

Media Module 711 (MM711) hardware vintages 3 through 6 require firmware vintages lower than V19, and any vintages greater than V19 are incompatible.  MM711 hardware vintage 7 requires firmware vintages greater than V20, but less than V57, and any other vintages are incompatible.  MM711 hardware vintage 20 and above requires firmware vintages V57 and greater, and any other vintages are incompatible.

The firmware filename structure for the MM711 shows hardware designation since there is a hardware dependency on the firmware.  For example, the filename MM711h7v21.fdl denotes the hardware vintage via h7 and v21 maps to the firmware vintage. 

Telephone Software Dependencies

The CM 3.0.1 features Security of IP Telephone Registration/H.323 Signaling Channel and Emergency Calls from Unnamed IP Endpoints require IP telephone software R2.3 or later. IP telephone software R2.3 or later requires TN799C hardware vintage 3 or later circuit packs. 

The 4606, 4612, and 4624 IP terminal endpoints do not support 2.0 and later firmware.

The 4630 and 4630SW IP terminal endpoints do no support 2.1 and later firmware.

The 4690 IP terminal endpoints require 1.7.5 or later firmware to be compatible with Communication Manager 2.1.

4630/4630SW IP Telephone R2.0 and R2.0.1 firmware is not compatible with server software Releases 10 and earlier.

Circuit Pack Firmware Compatibility

IP Interface Circuit Pack Firmware Available on the Communication Manager 3.x Software CD

The latest circuit pack firmware available at the time of launch is on the Communication Manager software CD. However, it is possible that the firmware on the CD is out-of-date, so the support web pages should always be checked for the latest firmware.

For TN circuit packs, the Software/Firmware Compatibility Matrix contained in this document lists the latest firmware available when this document is published for Communication Manager 2.0 and later releases. For releases earlier than Communication Manager 2.0, the matrix shows the latest firmware available at the time the specific CM release was launched. 

Avaya’s support web site should be checked for the latest firmware available for the following programmable circuit packs:

· TN2214CP

· TN2224CP

· TN2302AP for hardware vintages 3 and 11

· TN2312AP and BP

· TN2313AP

· TN2464BP

· TN2501AP

· TN2602AP

· TN464GP

· TN711DP

· TN799DP 

For firmware downloadable (programmable) circuit packs, the latest firmware available for the circuit pack should be used regardless of the release of Communication Manager software being used.  However, it is not necessary to update firmware that is working well unless there is a compelling reason to do so.

Whenever firmware is updated, the readme file should be checked for specific Communication Manager compatibility issues. The latest IP Interface circuit pack firmware, readme files, and instructions for installing downloadable firmware are found at the links given below:

The Latest Firmware for all Programmable TN Circuit Packs:
http://support.avaya.com/japple/css/japple?PAGE=Document&temp.bucketID=108025&temp.productID=107602&temp.documentID=200575&temp.releaseID=129470
TN2214CP:

http://support.avaya.com/japple/css/japple?PAGE=Document&temp.productID=129429&temp.bucketID=108025&temp.documentID=216072&temp.selectedRelease=129430#TN2214
TN2224CP:

http://support.avaya.com/japple/css/japple?PAGE=Document&temp.productID=129429&temp.bucketID=108025&temp.documentID=216054&temp.selectedRelease=129430#TN2224
TN2302AP:

http://support.avaya.com/japple/css/japple?temp.documentID=145455&temp.productID=129452&temp.releaseID=212899&temp.bucketID=108025&PAGE=Document
TN2312AP:

http://support.avaya.com/japple/css/japple?temp.documentID=145470&temp.productID=129452&temp.releaseID=212899&temp.bucketID=108025&PAGE=Document
TN2313AP:

http://support.avaya.com/japple/css/japple?PAGE=Document&temp.productID=129429&temp.bucketID=108025&temp.documentID=159934&temp.selectedRelease=129430#TN2313
TN2464BP:

http://support.avaya.com/japple/css/japple?PAGE=Document&temp.productID=129429&temp.bucketID=108025&temp.documentID=159932&temp.selectedRelease=129430#TN2464
TN2501AP:

http://support.avaya.com/japple/css/japple?temp.documentID=145465&temp.productID=129452&temp.releaseID=212899&temp.bucketID=108025&PAGE=Document
TN2602AP:

http://support.avaya.com/japple/css/japple?temp.documentID=236667&temp.productID=136527&temp.releaseID=228560&temp.bucketID=108025&PAGE=Document#TN2602
TN464GP:

http://support.avaya.com/japple/css/japple?temp.documentID=159921&temp.productID=107602&temp.releaseID=129470&temp.bucketID=108025&PAGE=Document#TN464
TN771DP:

http://support.avaya.com/japple/css/japple?PAGE=Document&temp.productID=136527&temp.bucketID=108025&temp.documentID=161271&temp.selectedRelease=152264#TN771
TN799DP:

http://support.avaya.com/japple/css/japple?temp.documentID=145513&temp.productID=129452&temp.releaseID=212899&temp.bucketID=108025&PAGE=Document
Media Gateway Firmware Compatibility

In general, media gateway and media module firmware is compatible with the current major release of Communication Manager and the previous major release (also known as N–1 compatibility). For example, media gateway and media module firmware released with Communication Manager 3.0.1 is intended to be compatible with Communication Manager 3.x and 2.x software releases. However, in practice, this is not always the case. Therefore, the Media Gateway, LSP, and Primary Server Compatibility Issues for Current Releases, Gateway/Server/LSP Compatibility Matrix, and Software/Firmware Compatibility Matrix sections of this document should always be checked for any known compatibility issues.

The latest media gateway and media module firmware available at the time of launch is on the Communication Manager software CD. However, it is possible that the firmware on the CD is out-of-date, so the support web pages should always be checked for the latest firmware.
For firmware downloadable media gateways and media modules, the Software/Firmware Compatibility Matrix contained in this document lists the latest firmware available at the date this document is published for Communication Manager 2.0 and later releases. For releases earlier than Communication Manager 2.0, the matrix shows the latest firmware available at the time the specific CM release was launched. 

Whenever firmware is updated, the readme file should be checked for specific Communication Manager compatibility issues.  Avaya’s support web site should be checked for the latest firmware available for a given media module. The latest G250, G350, & G700 Media Gateway firmware, media module firmware, readme files, and instructions for installing media gateway firmware are found at the following link:

http://support.avaya.com/japple/css/japple?temp.documentID=145453&temp.productID=129452&temp.releaseID=212899&temp.bucketID=108025&PAGE=Document
Gateway/Server/LSP Compatibility Matrix

	YES=Compatible

NO=Not Compatible
	Primary Media Server on 2.x Software
	Primary Media Server on 3.x Software
	LSP on 2.x Software
	LSP on 3.x Software

	23.xx.x Gateway Firmware
	YES
	NO
	YES
	NO

	24.xx.x Gateway Firmware
	G700 = NO

G250 = NA/NO

G350 = YES
	YES
	G700 = NO

G250 = NA/NO

G350 = YES
	YES

	LSP on 2.x Software
	YES
	NO

	LSP on 3.x Software
	YES
	YES


Note: The following order of upgrading a distributed configuration with a primary server, media gateways, and LSPs should be followed:
1. Media Gateways (MGP and Media Modules).  The order in which the Media Gateway (MGP) and Media Modules (MM) are upgraded does not matter.

2. LSPs

3. Primary Media Servers

This means that if an LSP is to be upgraded to a new version of CM software, all Media Gateways supported by that LSP must be running the appropriate new vintages of media gateway firmware. It also means that before a primary server is upgraded to a new release of CM software, all media gateways, media gateway media modules, and LSPs in the configuration must be upgraded appropriately before an upgrade of the primary server is attempted.

The Latest Firmware for G250, G350, and G700 Media Gateways and Media Modules:
http://support.avaya.com/japple/css/japple?PAGE=Document&temp.bucketID=108025&temp.productID=107602&temp.documentID=200575&temp.releaseID=129470
Telephone Software Compatibility

For telephones that support software downloads, the latest telephone software available for the specific phone should be used regardless of the release of Communication Manager software being used.  However, it is not necessary or desirable to update telephone software that is working well, unless the Communication Manager software is upgraded. Whenever telephone software is updated, the readme file should be checked for specific Communication Manager compatibility issues.

Some of the links below contain multiple vintages of telephone software for a given telephone.  The most recent (highest numbered) telephone software should always be used (e.g., R1.8.1 is more recent than R1.8).  For some telephones the latest telephone software available is marked as “Current” on the web pages that are linked below.  For each telephone, the most recent telephone software available when this document was issued is listed.  The links should always be checked to determine if more recent telephone software is available:

2400 Digital Telephones:

Release 4 for the 2420 and release 2 for the 2410 were the most recent releases when this document was issued.  Always check for more recent firmware.
http://support.avaya.com/japple/css/japple?PAGE=ProductArea&temp.productID=130048&temp.bucketID=108025
All 46xx IP Telephones:

http://support.avaya.com/japple/css/japple?PAGE=ProductArea&temp.productID=107755&temp.bucketID=108025
6400 Digital Telephones:

http://support.avaya.com/japple/css/japple?PAGE=ProductArea&temp.productID=107751&temp.bucketID=108025
8400 Digital Telephones:

http://support.avaya.com/japple/css/japple?PAGE=ProductArea&temp.productID=107760&temp.bucketID=108025
IP Agent:

R6 was the most recent when this document was issued.  Always check for more recent firmware.
http://support.avaya.com/japple/css/japple?PAGE=ProductArea&temp.productID=107698&temp.bucketID=108025
 IP Softphone:

R5.2 was the most recent when this document was issued.  Always check for more recent firmware.

http://support.avaya.com/japple/css/japple?PAGE=ProductArea&temp.productID=107767&temp.bucketID=108025
IP Softphone for Pocket PC:

R2.3.1 was the most recent when this document was issued.  Always check for more recent firmware.
http://support.avaya.com/japple/css/japple?PAGE=ProductArea&temp.productID=135860&temp.bucketID=108025
Softconsole:

R1.5.3 was the most recent when this document was issued.  Always check for more recent firmware.
http://support.avaya.com/japple/css/japple?PAGE=ProductArea&temp.productID=130009&temp.bucketID=108025
Service Pack Compatibility

Please refer to section Service Packs for Communication Manager 3.x Releases for specific Communication Manager 3.x details concerning service packs.

The S8300, S8500, and S87x0 Media Servers have Communication Manager service packs available for specific software releases.  These service packs contain fixes for known issues and the latest service pack for the appropriate release must be applied during initial installation.  Furthermore, if a service pack becomes available for a given release after installation of a media server has been completed, the service pack must be applied to bring Communication Manager software running on the server up-to-date.  Service Packs are release dependent so it is extremely important to be sure the service pack you are applying is for the appropriate software release.

Be aware that a new service pack might become available on the support.avaya.com web pages, so it is best to subscribe for automated PCN e-mail notifications or check the link directly below for the latest service pack for a specific Communication Manager software release:

Service Packs for all releases:

http://support.avaya.com/japple/css/japple?PAGE=Document&temp.bucketID=108025&temp.productID=107602&temp.documentID=200575&temp.releaseID=129470
Communication Manager 1.3:

The latest Communication Manager 1.3 (software release R011x.03.0.526.5 / SALT 318.5 launched May 2003 in the U.S.) software update is available at the following link:  

http://support.avaya.com/japple/css/japple?temp.documentID=152745&temp.productID=129452&temp.releaseID=152297&temp.bucketID=108025&PAGE=Document
Communication Manager 1.3.1:

The latest Communication Manager 1.3.1 (software release R011x.03.1.531.0 / SALT 323.0 launched September 2003 in the U.S.) software update is available at the following link:  

http://support.avaya.com/japple/css/japple?temp.documentID=156493&temp.productID=129452&temp.releaseID=156501&temp.bucketID=108025&PAGE=Document
Communication Manager 1.3.2:

The latest Communication Manager 1.3.2 (software release R011x.03.2.536.1 / SALT 328.1 launched May 2004 in the U.S.) software update is available at the following link:  

http://support.avaya.com/japple/css/japple?temp.documentID=162758&temp.productID=129452&temp.releaseID=162131&temp.bucketID=108025&PAGE=Document
Communication Manager 2.0:

The latest Communication Manager 2.0 (software release R012x.00.0.219.0 / Offer 219.0 launched December 8th 2003 in the U.S.) software update is available at the following link:  

http://support.avaya.com/japple/css/japple?temp.documentID=157671&temp.productID=129452&temp.releaseID=156181&temp.bucketID=108025&PAGE=Document
Communication Manager 2.0.1:

The latest Communication Manager 2.0.1 (software release R012x.00.1.221.1 / Offer 221.1 launched February 9th 2004 in the U.S.) software update is available at the following link:

http://support.avaya.com/japple/css/japple?temp.documentID=159751&temp.productID=129452&temp.releaseID=159639&temp.bucketID=108025&PAGE=Document
Communication Manager 2.1:

The latest Communication Manager 2.1 (software release R012x.01.0.411.7 / Offer 411.7 launched July 12th 2004 in the U.S.) software update is available at the following link:

http://support.avaya.com/japple/css/japple?temp.documentID=195615&temp.productID=129452&temp.releaseID=162392&temp.bucketID=108025&PAGE=Document
Communication Manager 2.1.1:
The latest Communication Manager 2.1.1 (software release R012x.01.1.414.1 / Offer 414.1 launched September 7th 2004 in the U.S.) software update is available at the following link:

http://support.avaya.com/japple/css/japple?temp.documentID=201992&temp.productID=129452&temp.releaseID=201996&temp.bucketID=108025&PAGE=Document
Communication Manager 2.2:

Communication Manager 2.2 (software release R012x.02.0.111.4 / Offer 111.4 launched January 10th 2005 in the U.S.) service pack is available at the following link:

http://support.avaya.com/japple/css/japple?temp.documentID=226414&temp.productID=136527&temp.releaseID=212033&temp.bucketID=108025&PAGE=Document#2.2_svc_pack
Communication Manager 2.2.1:

Communication Manager 2.2.1 (software release R012x.02.1.118.1 launched November 7th 2005 in the U.S.) service pack is available at the following link: http://support.avaya.com/japple/css/japple?temp.documentID=278936&temp.productID=136527&temp.releaseID=212033&temp.bucketID=108025&PAGE=Document
Communication Manager 3.0:

Communication Manager 3.0 (software release R013x.00.0.340.3 launched June 13th 2005 in the U.S.) service pack is available at the following link:

http://support.avaya.com/japple/css/japple?temp.documentID=236672&temp.productID=136527&temp.releaseID=228560&temp.bucketID=108025&PAGE=Document
Communication Manager 3.0.1:

Communication Manager 3.0.1 (software release R013x.00.1.346.0 launched October 3rd 2005 in the U.S.) service pack is available at the following link: http://support.avaya.com/japple/css/japple?temp.documentID=276193&temp.productID=136527&temp.releaseID=228560&temp.bucketID=108025&PAGE=Document
E-Notifications: 

To be automatically notified via e-mail when new service packs and firmware updates are available, you can subscribe to receive all PCN notifications via My E-Notifications on support.avaya.com.

Integrated Management

The latest updates for Integrated Management applications can be accessed from the following links:

Fault and Performance Manager:

http://support.avaya.com/japple/css/japple?PAGE=ProductArea&temp.productID=139474&temp.bucketID=108025
MultiSite Administration:

http://support.avaya.com/japple/css/japple?PAGE=ProductArea&temp.productID=139470&temp.bucketID=108025
Network Management:

http://support.avaya.com/japple/css/japple?PAGE=ProductArea&temp.productID=158280&temp.bucketID=108025
Proxy Agent:

http://support.avaya.com/japple/css/japple?PAGE=ProductArea&temp.productID=139482&temp.bucketID=108025
Site Administration:

http://support.avaya.com/japple/css/japple?PAGE=ProductArea&temp.productID=139469&temp.bucketID=108025
Voice Announcement Manager:

http://support.avaya.com/japple/css/japple?PAGE=ProductArea&temp.productID=139476&temp.bucketID=108025
VoIP Monitoring Manager:

http://support.avaya.com/japple/css/japple?PAGE=ProductArea&temp.productID=139480&temp.bucketID=108025
Software/Firmware Compatibility Matrix

For Communication Manager releases prior to 2.0, the information presented in the matrix shows the circuit pack, gateway, and media module firmware available at the time of launch. For Communication Manager 2.0 and later releases, the information presented in the matrix shows the latest circuit pack, gateway, and media module firmware available and recommended for the specific release at the time this document is published. Due to the frequency that new firmware becomes available it is possible that this document is temporarily out of date. Therefore, the support.avaya.com web pages should always be checked for the very latest software and firmware available.  

Communication Manager 2.0 and later releases are not available for the DEFINITY Server R.

Communication Manager 2.1 and later releases are not available for the S8100 Media Server.

Communication Manager 3.0 and later releases are not available for the DEFINITY Server SI.

Communication Manager 2.1 and later releases do not install or run on a S8300A Media Server.  An S8300B Media Server is required to run Communication Manager 2.1 and later releases.

Communication Manager 2.2 cannot be installed on a S8300B Media Server if the hard drive is loaded with Remaster Program (RP) 3-11 software and TFTP via the service port is the installation method used.

Links to the latest TN circuit pack firmware, media gateway firmware, media module firmware, and Communication Manager service packs can be found at the following link:

http://support.avaya.com/japple/css/japple?PAGE=Document&temp.bucketID=108025&temp.productID=107602&temp.documentID=200575&temp.releaseID=129470
	Communication Manager Software / Firmware Compatibility Matrix

	S8100 MEDIA SERVER

	CM

Release
	Launched Load
	TN799DP

(C-LAN)

FW
	TN2302AP (Prowler)

FW
	TN2501AP (VAL)

FW

	CM 1.1
	Not Available
	Not Available
	Not Available
	Not Available

	CM 1.1.2
	Not Available
	Not Available
	Not Available
	Not Available

	CM 1.1.3
	Not Available
	Not Available
	Not Available
	Not Available

	CM 1.2


	R011c.02.0.110.4

Offer S8100-MV1.2-110.4
	See TN799DP: Link
	See TN2302AP: Link
	See TN2501AP: Link

	CM 1.3
	R011c.03.0.526.5 Offer S8100-CM1.3-526.5
	See TN799DP: Link
	See TN2302AP: Link
	See TN2501AP: Link

	CM 1.3.1
	R011c.03.1.531.0

Offer S8100-1.3.1-531.0
	See TN799DP: Link
	See TN2302AP: Link
	See TN2501AP: Link

	CM 1.3.2
	Not Available
	Not Available
	Not Available
	Not Available

	CM 2.0
	R012c.00.0.219.0

Offer S8100-2.0-219.0
	V15 or later

See TN799DP: Link
	HV3-6: V93 or later

HV11+: V107 or later

See TN2302AP: Link
	V9 or later

See TN2501AP: Link

	CM 2.0.1
	R012c.00.1.222.0 Offer S8100-2.0.1-222.0
	V15 or later

See TN799DP: Link
	HV3-6: V93 or later

HV11+: V107 or later

See TN2302AP: Link
	V9 or later

See TN2501AP: Link

	CM 2.1 and Later Releases
	Not Available

	G150 MEDIA GATEWAY COMPATIBILITY

	CM

Release
	G150 Media Gateway Version

	CM 2.2 & 2.2.1
	3.0(100)

	CM 3.0 & 3.0.1
	3.0(100) & 3.0(101)

	G250 (MGP only), G350, & G700 MEDIA GATEWAY COMMON FIRMWARE

	CM Release
	G250 G350 G700 FW
	MM 710  FW
	MM 711 FW
	MM 712 FW


	MM 717 FW


	MM 720 FW
	MM 760 FW
	P330 FW  DM

	CM 1.1
	V3

G250: NA
	V3
	V4
	V2
	N/A
	N/A
	V3
	3.9.0

	CM 1.1.2
	V5

G250: NA
	V4
	V5
	V2
	N/A
	N/A
	V4
	3.12.1

	CM 1.1.3
	V5

G250: NA
	V4
	V5
	V2
	N/A
	N/A
	V4
	3.12.1

	CM 1.2
	V8

G250: NA
	V5
	V12
	V3
	N/A
	V1
	V8
	3.12.1

	CM 1.3
	V20.13

G250: NA
	V8
	V16
	V5
	N/A
	V3
	V11
	3.12.1

	CM 1.3.1
	V20.17

G250: NA
	V8
	HV3-6: V16

HV7: V21
	V5
	N/A
	V3
	V16
	4.0.17

	CM 1.3.2
	V21.24.0

G250: NA
	V8
	HV3-6: V17

HV7: V22

HV20+: V58
	V5
	N/A
	V3
	V24
	4.0.17

4.0.4

	CM 2.0
	G700: V23.23.0

G350: V24.21.1 

G250: NA
	V13
	HV3-6: V17

HV7: V24

HV20+:V67
	V7
	N/A
	V6
	V57
	4.1.6

4.6.6

	CM 2.0.1
	G700: V23.23.0

G350: V24.21.1 

G250: NA
	V13
	HV3-6: V17

HV7: V24

HV20+:V67
	V7
	N/A
	V6
	V57
	4.1.6

4.6.6

	CM 2.1
	G700: V23.23.0

G350: V24.21.1 

G250: NA
	V13
	HV3-6: V17

HV7: V24

HV20+:V67
	V7
	V4
	V6
	V57
	4.1.6

4.6.6

	CM 2.1.1
	G700: V23.23.0

G350: V24.21.1 

G250: NA
	V13
	HV3-6: V17

HV7: V24

HV20+:V67
	V7
	V4
	V6
	V57
	4.1.6

4.6.6

	CM 2.2
	G700: V23.23.0

G350: V24.21.1 

G250: NA
	V13
	HV3-6: V17

HV7: V24

HV20+:V67
	V7
	V4
	V6
	V57
	4.1.6

4.6.6

	CM 2.2.1
	G700: V23.23.0

G350: V24.21.1 

G250: NA
	V13
	HV3-6: V17

HV7: V24

HV20+:V67
	V7
	V4
	V6
	V57
	4.1.6

4.6.6

	CM 3.0
	V24.21.1


	V13
	HV3-6: V17

HV7: V24

HV20+:V67
	V7
	 V4
	V6
	V57
	4.1.6

4.6.6

	CM 3.0.1
	V24.21.1
	V13
	HV3-6: V17

HV7: V24

HV20+:V67
	V7
	 V4
	V6
	V57
	4.1.6

4.6.6

	G350 MEDIA GATEWAY SPECIFIC FIRMWARE

	CM Release
	G350 Analog FW
	MM312 FW
	MM714 FW
	MM722 FW

	CM 2.0
	V67
	V4
	V67
	V2

	CM 2.0.1
	V67
	V4
	V67
	V2

	CM 2.1
	V67
	V4
	V67
	V2

	CM 2.1.1
	V67
	V4
	V67
	V2

	CM 2.2
	V67
	V4
	V67
	V2

	CM 2.2.1
	V67
	V4
	V67
	V2

	CM 3.0 
	V67
	V4
	V67
	V2

	CM 3.0.1
	V67
	V4
	V67
	V2

	S8300, S8500, & S8700 SERIES MEDIA SERVERS

	CM Release
	Launched Load
	Latest Service Pack
	TN799

(C-LAN)

FW
	TN2302 (Cruiser)

FW
	TN2312 (IPSI)

FW
	TN2501 (VAL) FW
	TN2602 (IP 320) FW

	CM 1.1
	R011x.01.0.060.4
	01.0.060.4-0016.tar.gz
	See TN799DP: Link 
	See TN2302AP: Link 
	See TN2312AP: Link
	See TN2501AP: Link
	Not Available

	CM 1.1.2
	R011x.01.2.065.3
	01.2.065.3-0005.tar.gz
	See TN799DP: Link
	See TN2302AP: Link
	See TN2312AP: Link
	See TN2501AP: Link
	Not Available

	CM 1.1.3
	R011x.01.3.065.5
	01.3.065.5-0001.tar.gz
	See TN799DP: Link
	See TN2302AP: Link
	See TN2312AP: Link
	See TN2501AP: Link
	Not Available

	CM 1.2
	R011x.02.0.110.4


	02.0.110.4-5619.tar.gz
	See TN799DP: Link
	See TN2302AP: Link
	See TN2312AP: Link
	See TN2501AP: Link
	Not Available

	CM 1.3
	R011x.03.0.526.5


	03.0.526.5-5767.tar.gz
	See TN799DP: Link
	See TN2302AP: Link
	See TN2312AP: Link
	See TN2501AP: Link
	Not Available

	CM 1.3.1
	R011x.03.1.531.0
	See

Communication Manager 1.3.1:
Link
	See TN799DP: Link
	See TN2302AP: Link
	See TN2312AP: Link
	See TN2501AP: Link
	Not Available

	CM 1.3.2
	R011x.03.2.536.1
	See

Communication Manager 1.3.2: 

Link
	See TN799DP: Link
	See TN2302AP: Link
	See TN2312AP: Link
	See TN2501AP: Link
	Not Available

	CM 2.0
	R012x.00.0.219.0
	See
Communication Manager 2.0:
Link
	V15 or later

See TN799DP: Link
	HV3-6: V93 or later

HV11+: V107 or later

See TN2302AP: Link
	V22 or later

See TN2312AP: Link
	V9 or later

See TN2501AP: Link
	Not Available

	CM 2.0.1
	R012x.00.1.221.1
	See
Communication Manager 2.0.1:
Link
	V15 or later

See TN799DP: Link
	HV3-6: V93 or later

HV11+: V107 or later

See TN2302AP: Link
	V22 or later

See TN2312AP: Link
	V9 or later

See TN2501AP: Link
	Not Available

	CM 2.1
	R012x.01.0.411.7
	See

Communication Manager 2.1:
Link
	V15 or later

See TN799DP: Link
	HV3-6: V93 or later

HV11+: V107 or later

See TN2302AP: Link
	V22 or later

See TN2312AP: Link
	V9 or later

See TN2501AP: Link
	Not Available

	CM 2.1.1
	R012x.01.1.414.1
	See

Communication Manager 2.1.1:
Link
	V15 or later

See TN799DP: Link
	HV3-6: V93 or later

HV11+: V107 or later

See TN2302AP: Link
	V22 or later

See TN2312AP: Link
	V9 or later

See TN2501AP: Link
	Not Available

	CM 2.2
	R012x.02.0.111.4
	See

Communication Manager 2.2:
Link
	V15 or later

See TN799DP: Link
	HV3-6: V93 or later

HV11+: V107 or later

See TN2302AP: Link
	V22 or later

See TN2312AP: Link
	V9 or later

See TN2501AP: Link
	Not Available

	CM 2.2.1
	R012x.02.1.118.1
	See

Communication Manager 2.2.1:
Link
	V15 or later

See TN799DP: Link
	HV3-6: V93 or later

HV11+: V107 or later

See TN2302AP: Link
	V22 or later

See TN2312AP: Link
	V9 or later

See TN2501AP: Link
	Not Available

	CM 3.0 
	R013x.00.0.340.3
	See

Communication Manager 3.0:
Link
	V15 or later

See TN799DP: Link
	HV3-6: V93 or later

HV11+: V107 or later

See TN2302AP: Link
	V22 or later

See TN2312AP: Link
	V9 or later

See TN2501AP: Link
	V7 or later

See TN2602AP: Link

	CM 3.0.1 
	R013x.00.1.346.0
	See

Communication Manager 3.0.1:
Link
	V15 or later

See TN799DP: Link
	HV3-6: V93 or later

HV11+: V107 or later

See TN2302AP: Link
	V22 or later

See TN2312AP: Link
	V9 or later

See TN2501AP: Link
	V7 or later

See TN2602AP: Link

	DEFINITY SERVER SI / CSI / R



	CM Release
	Launched Load
	TN799DP

(C-LAN)

FW
	TN2302AP (Cruiser)

FW
	TN2501AP

(VAL)

FW

	CM 1.1
	R011r.01.0.60.4

R011i.01.0.60.4
	See TN799DP: Link
	See TN2302AP: Link
	See TN2501AP: Link

	CM 1.1.2
	R011r.01.2.65.3

R011i.01.2.65.3
	See TN799DP: Link
	See TN2302AP: Link
	See TN2501AP: Link

	CM 1.1.3
	R011r.01.3.65.4

R011i.01.3.65.4
	See TN799DP: Link
	See TN2302AP: Link
	See TN2501AP: Link

	CM 1.2
	R011r.02.0.110.4

R011i.02.0.110.4
	See TN799DP: Link
	See TN2302AP: Link
	See TN2501AP: Link

	CM 1.3
	R011r.03.0.526.5

R011i.03.0.526.5
	See TN799DP: Link
	See TN2302AP: Link
	See TN2501AP: Link

	CM 1.3.1
	R011r.03.1.531.0

R011i.03.1.531.0
	See TN799DP: Link
	See TN2302AP: Link
	See TN2501AP: Link

	CM 1.3.2
	R011r.03.2.536.1

R011i.03.2.536.1
	See TN799DP: Link
	See TN2302AP: Link
	See TN2501AP: Link

	CM 2.0
	Not available for R

R012i.00.0.219.0
	V15 or later 

See TN799DP: Link
	HV3-6: V93 or later

HV11+: V107 or later

See TN2302AP: Link
	V9 or later

See TN2501AP: Link

	CM 2.0.1
	Not available for R

R012i.00.1.221.1
	V15 or later 

See TN799DP: Link
	HV3-6: V93 or later

HV11+: V107 or later

See TN2302AP: Link
	V9 or later

See TN2501AP: Link

	CM 2.1
	Not available for R

R012i.01.0.411.7
	V15 or later

See TN799DP: Link
	HV3-6: V93 or later

HV11+: V107 or later

See TN2302AP: Link
	V9 or later

See TN2501AP: Link

	CM 2.1.1
	Not available for R

R012i.01.1.414.1
	V15 or later

See TN799DP: Link
	HV3-6: V93 or later

HV11+: V107 or later

See TN2302AP: Link
	V9 or later

See TN2501AP: Link

	CM 2.2
	Not available for R

R012i.02.0.111.4
	V15 or later

See TN799DP: Link
	HV3-6: V93 or later

HV11+: V107 or later

See TN2302AP: Link
	V9 or later

See TN2501AP: Link

	CM 2.2.1
	Not available for R

R012i.02.1.118.1
	V15 or later

See TN799DP: Link
	HV3-6: V93 or later

HV11+: V107 or later

See TN2302AP: Link
	V9 or later

See TN2501AP: Link

	CM 3.0 
	Not available for R

Not available for SI

R012i.00.0.340.3
	V15 or later

See TN799DP: Link
	HV3-6: V93 or later

HV11+: V107 or later

See TN2302AP: Link
	V9 or later

See TN2501AP: Link

	CM 3.0.1 
	Not available for R

Not available for SI

R012i.00.1.346.0
	V15 or later

See TN799DP: Link
	HV3-6: V93 or later

HV11+: V107 or later

See TN2302AP: Link
	V9 or later

See TN2501AP: Link








Document Description


WARNING: The links included in this document might not work with Netscape browsers.





This document describes the latest service packs and firmware versions needed for 2.x releases of Communication Manager software.





The intent is for this document to be as complete and accurate as possible.  However, due to the ever-changing nature of some of this information the document must be re-released as new information becomes available.  For this reason, it is important to check for later versions of the document periodically.  The changes between versions of the document are listed and can be used as a quick reference as to what has been updated since the previous version.





This document contains information pertaining to Communication Manager 3.x only.  However, the upgrade and matrix portion of the document contains information pertaining to all releases (e.g., 1.x, 2.x, 3.x, etc.) of Communication Manager for completeness.  For detailed information on 2.x releases of Communication Manager please refer to version 6 of the Communication Manager 2.x Software & Firmware Compatibility Matrix, which is referenced in section � REF _Ref103410552 \h � \* MERGEFORMAT �Communication Manager 2.x Releases� of this document.


Version 1


This version of the document is the initial version supporting release 3.0 of Communication Manager software. Communication Manager 3.0 is available June 13th, 2005. 


Version 2


This version of the document includes updates to the Communication Manager 3.x upgrades sections.


Communication Manager 2.2.1, available November 7th, 2005, has been added.


Communication Manager 3.0.1, available October 3rd, 2005, has been added.


The G150 Media Gateway has been added.


Media Gateway, TN circuit pack, and Communication Manager compatibility has been clarified. The compatibility matrix has been updated to show the latest firmware available and recommended at the date this document is published for Communication Manager releases 2.0 and later. For Communication Manager releases earlier than 2.0, the matrix still shows the latest firmware available at the time of launch.


An erroneous statement concerning 4602 telephone and CM 3.0 compatibility has been removed. 




















1
©2005 Avaya Inc. All Rights Reserved. Avaya and the Avaya logo are trademarks of Avaya Inc. and may be registered in certain jurisdictions. All trademarks identified by ® and ™ are registered trademarks or trademarks respectively, of Avaya Inc. All other registered trademarks or trademarks are property of their respective owners.

BDH

4
©2005 Avaya Inc. All Rights Reserved. Avaya and the Avaya logo are trademarks of Avaya Inc. and may be registered in certain jurisdictions. All trademarks identified by ® and ™ are registered trademarks or trademarks respectively, of Avaya Inc. All other registered trademarks or trademarks are property of their respective owners.

BDH


[image: image2.png]_1083589839.doc
[image: image1.png]AVAYA







