Configuration Note 4052 - Ver B (3/01)

Octel 350

Message Server

ROCKWELL SPECTRUM ACD

## 1.0  METHOD OF INTEGRATION

With RS232 integration, call information is transmitted over a digital RS232 link between the Spectrum ACD and the Octel voice server.  Voice communications are provided by as separate path created by analog lines on the Spectrum ACD that connect to the Octel voice server.  When an incoming call is received, it is accompanied by a digital message from the Spectrum ACD containing call information.  The Octel voice server then answers the call and plays the appropriate greeting.  To set or cancel message-waiting notification, the Octel voice server sends a digital message over the RS232 link to the Spectrum ACD.

With RS232 integration, call information is transmitted over a digital RS232 link between the Spectrum ACD and the Octel voice server

## 2.0  Octel ORDERING INFORMATION

•	ATIC’s

Octel ordering information

•	SMDI Integration kit

•	Disk Drives

## 3.0 Rockwell Spectrum  HARDWARE REQUIREMENTS

•	Loop- start or ground-start line appearances, one per Octel port

Rockwell Spectrum ACD hardware requirements

•	One single-line channel for remote service access

•	25-pair, male Amphenol cables, one per ATIC

•	PDA 3 I/O card

3.1  Rockwell Spectrum SOFTWARE REQUIREMENTS

Rockwell Spectrum ACD software requirements

•	Minimum software level - Spectrum 5.0

## 4.0  SUPPORTED integration FEATURES

•	Station forward to personal greeting
- all calls	(including calls when the agent is not signed into the ACD 		  application)

Supported integration features

•	System forward to personal greeting
- busy
- no answer

•	Multiple Greetings

- busy
- no answer

•	Message Waiting Indicator
-  lights

•	Automated Attendant

•	Outcalling

•	Call Sender

•	Multiple Return-to-operator

•	Direct Call

•	Personal greeting of original-called party on a multiple-call forward

•	Reply to message left by subscriber in internal telephone-answering mode.

## 5.0	Configuring the Rockwell Spectrum ACD to 	Integrate

Create the data link for the voice server;

ENT-SDP	(Devtype = vmail)

o	Create the application telescript for integrating to the voice server;

Configuring the Rockwell Spectrum ACD to integrate

ENT-VSTEP-APPL   (VMAIL DATA LINK)

o	Create the application for integrating to the voice server;

ENT-APPL 	(Routing => Applications)

o	Define Class of Service for voice server ports

ENT-COS	(System => Class of Service)

o	Create a voice server agent group;

ENT-AGRP	(Staffing => Personnel)

o	Provision the circuits to the voice server and assign them to the voice 	server agent group;

ENT-EQPT = AIC		(System => Configure => Equip. 						Cards)

ENT-CTPR  =   FXSL	(System => Configure => Custom 						Timing Profile)

ENT-TO			(define low level port 								characteristics)

o	Provision line termination’s.

ENT-DEV

o	Subscribers should call forward their phones to the voice server 	application pilot number.  System call rerouting may also be used. 	Simply reroute extensions to the voice server application pilot 	number.

o	The decibel level of the lines coming into the Octel 350 should be            	-5.5 dB or higher or call volume might be insufficient.

## 6.0  Configuring the Octel 350 Platform

¨	Menu 1.1  -  System Parameters

§	Company Phone Number (include area code):

§	ACP Integration Used?:   (required if more than 3 SMDI links used)

§	Type of Switch connected to:  3 - 1AESS Full Duplex  SMDI

§	Number of Digits in Extension used for Outcalling and ECP: 4

§	Number of Digits in Extension used for Message Waiting:  7

§	Sender ID Used for Telephone Answering Messages:  2  - Sender ID if Known

¨	Menu 4.1  -  Port Assignments

§	Extension/Phone No.:  It is important that this column must contain the nine digit number associated with each link defined as follows:  LLGGGMMMM

where,

LL		=	physical link number (01-02)

Configuring the Octel 350 platform

GGG		=	message desk number (001-999)

MMMM	=	terminal number (0001-2047)

The physical link number corresponds with the actual connector on the distribution board used to make the connection.  See Table 3.

The message desk number and terminal number are assigned to each line by the Spectrum ACD.

All nine digits must be entered.  Each entry must be unique.  The use of the physical link number, however, allows redundant message desk/terminal number coordinates to be used as long as they are spread across separate links.

Example:  An Octel 350 system is being integrated to two Spectrum ACD switches.  Switch #1 has a 6-member multiline hunt group connecting to Octel 350 ports 1A through 1F, and the SMDI link connects to distribution board connector J44.  Switch #2 has a 6-member multiline hunt group connecting to Octel 350 ports 1G through 1L, and the SMDI link connects to distribution board connector J43.  Both groups use the same message desk number (001).

See Table 4 for examples of Menu 4.1 entries.

If the port configuration is ever changed (e.g., additional ports and/or SMDI links added), the appropriate fields in Menu 4.1 must be updated.

Configuring the Octel 350 platform

§	Line Type: 52  ground start trunks
		 35  loop start trunks

§	M column: N

¨	Menu 4.2  -  RS-232 Message Waiting

§	Special RS-232 Message Waiting:  Y
This parameter enables message waiting utilizing the background task instead of the line task.

¨	Menu 6.1  -  Transfer Dialing Sequences

§	“Flash” On-hook Time:  500  milliseconds
§	“Pause” Time:  1200  milliseconds
§	Dialing Sequence to Transfer a Call:  FPN
§	Dialing Sequence to Reconnect with a Call -
					Ring/No Answer: F
						Busy:  F

NOTE:  Verify these dialing sequences for each installation.

o	Menu 6.5  -  Integration Link Management

§	Link Number:

§	Link Name:
	Type of Switch to which the System is Integrated: 3 - 1A ESS/SMDI, 	Full Duplex
	Baud Rate: 3 - 1200 Baud
	Number of Data Bits:  0 - 7 Data Bits
	Number of Stop Bits:  0 - 1 Stop Bits
	Parity (None/Odd/Even): 2 - Even Parity
	XON/XOFF: 1 - Ignore XON/XOFF
	Carrier Detect: 1 - Carrier Detect Not Used
	Extension to Check SMDI Message Waiting: 5551212
	Max. Msg. Waiting Operations per second: 1
	Switch Number to which this Link is Associated:   
	SMDI link down alarm threshold counter:  1

Configuring the Octel 350 platform

The number entered in the Switch Number field must also be entered as the link number in the Extension/Phone No. column of Menu 4.1.

¨	Menu 8/9.1  -  Subscriber Mailbox Profile

•	“Subscriber’s Extension Number”:  If the subscriber is allowed to have message waiting, enter all seven digits of his or her phone number in this entry.

•	“SMDI Link Number”:  Enter the physical link number of the SMDI group which is associated with the subscriber’s Spectrum ACD switch.

7.0  Connecting the Spectrum SMDI link to the octel                      	350

Connecting the SMDI link to the Octel 350

o	Physically connect the SMDI links from the Spectrum ACD to the 	Octel 350 system using the data cables provided by Lucent OMD.  	Connect each cable to the distribution board located on the back of 	the Octel 350.  The end of the cable with the right angle DB-25 	connector is connected to the distribution board.  See Figure 9.

o	Up to three SMDI links can be directly connected to the distribution 	board.  The links are connected to connector designations J44 (Octel 	Link 1), J43 (Octel Link 2), and J46 (Octel Link 3).

## 7.1  Connecting Analog Voice Channels

Each Analog TIC supports 12 ports.  Each Analog TIC connects to the switch via a single cable.  The first port uses the first pair of connectors (1,26), the second port uses the second pair (2,27), etc.

Connecting the analog channels

o	Physically connect the lines used for the voice path between the 	switch and the Octel 350.  The voice lines are connected to 	the 	switch using customer-supplied cables which terminate on the 	main backplanes on the Octel 350 frames.  See Figures 1 and 2.

## 7.2  Testing the Installation

o	Create two Octel 350 mailboxes associated with two test 	extensions.  Record a name and personal greeting for each mailbox.

Testing the installation when completed

o	System forward one test extension to the Octel 350’s System 	Access Number.

o	Using one test extension, call the other test extension.  You should 	hear 	the personal greeting.

o	Leave a message.  Verify that message waiting indication turns on.

o	Verify that return-to-operator works properly.

o	Call the Octel 350 from a test extension.  You should 	immediately hear the recorded name and be asked to enter your 	password.

o	Review the message in the mailbox.  Verify the direct reply feature 	works by pressing 8 at the end of the message.

o	Delete the message.  Verify that message waiting indication turns off.

## 8.0  Considerations

Important notes regarding this integration

8.1	Ground-start lines provide immediate disconnect and are recommended.  If loop-start signaling is used, subscribers might experience dialtone at the end of messages and ports might take longer to disconnect if a 600 ms open is not provided.

©2001 AVAYA Inc. All rights reserved. All trademarks identified by the ®, SM and TM are registered trademarks, servicemarks or trademarks respectively. All other trademarks are properties of their respective owners. The above information is based on knowledge available at the time of publication and is subject to change without notice. Printed in U.S.A.

AVAYA Inc.

1001 Murphy Ranch Road

Milpitas, CA 95035-7912

(408) 321-2000

http://www.avaya.com