Avaya Gateway Installation Wizard Job Aid and Preinstallation Worksheet ##### Avaya G700 Media Gateway controlled by an Avaya S8300 Media Server or S8700 Media Server

The Avaya Gateway Installation Wizard (GIW) supports G700 Media Gateways that are controlled by an S8700 Media Server but do not have a Local Survivable Processor (LSP).  The GIW allows you to automatically configure the G700 Media Gateway IP addresses only. For all other tasks associated with a G700 installation, you must follow the instructions in the Installation and Upgrades for G700 with S8300 or S8700, 555-234-100, Issue 3. See the checklist of tasks on the G700 Installation Checklist later in this document.

Additionally, you cannot configure an X300 Expansion Module with the GIW.  Instead, use the instructions in the Installation and Upgrades for G700 with S8300 or S8700, 555-234-100, Issue 3.

CAUTION: If the G700 Media Gateway is supported by an S8300 primary controller or a Local Survivable Processor (LSP), do not use the GIW. Instead, use the Avaya Installation Wizard (AIW), which also allows you to configure the G700 Media Gateway and its components, as well as to install the S8300 Media Server or LSP.

The following activities must take place before you can install an Avaya G700 Media Gateway:

- A meeting is held with the customer to complete the Preinstallation Worksheet in the following pages of this document. The Preinstallation Worksheet contains specific information about the customer’s network configuration that is required to use the Avaya Gateway Installation Wizard to configure the G700 Media Gateway.

- A TFTP server must be established either on your laptop or on a server connected to the LAN to which the G700 Media Gateway will connect. If your laptop is used, the customer must assign it a static IP address within the customer LAN and an Ethernet port provided within a few feet of the G700 Media Gateway.

- The most recent versions of firmware are retrieved from the support.avaya.com Web site or from a CD-ROM shipped either from the factory or from your services support team. The firmware updates must be downloaded onto your laptop PC you are using for the Avaya Gateway Installation Wizard.

- The Avaya Gateway Installation Wizard must be downloaded to your laptop P.C.

# G700 Installation Checklist

The tasks to install a G700 Media Gateway and the personnel who normally perform the tasks are summarized in the following table:

| Task | Who Does It |
| --- | --- |
| Step 1: Complete the Preinstallation Worksheet Step 1a: Get Stack Master Information Step 1b: Get Media Gateway Information | Project Manager and Customer LAN Administrator |
| Step 2: Download the Avaya Gateway Installation Wizard | Installer |
| Step 3: Set up a TFTP Server, If Necessary, and Download Firmware to a TFTP Server | Installer and Customer LAN Administrator |
| Step 4: Install Hardware for the G700 Media Gateway | Installer |
| Step 5: Run the Avaya Gateway Installation Wizard | Installer, using the Preinstallation Worksheet |
| Step 6: Prepare to Install the Firmware on the G700 | Installer |
| Step 7: Install New Firmware on the G700 Media Gateway | Installer |
| Step 8: Administer Communication Manager on the S8700 Primary Controller | Installer |
| Step 9: Complete the Installation | Installer |
| Step 10: Register Warranty | Installer, using information from customer |

# Step 1: Complete the Preinstallation Worksheet

Complete this worksheet with the assistance of the customer’s corporate local area network (LAN) administrator.

Allow up to 1 hour to complete this worksheet.

CAUTION: If you do not have a completed document, DO NOT BEGIN THE SERVER CONFIGURATION. DO NOT GUESS AT THESE VALUES. If you use the wrong values, you can corrupt the customer's network.

Note: The customer’s LAN administrator might require you to change IP addresses to prevent conflicts with existing endpoints on the corporate LAN. Make precise notes of any changes and follow the LAN administrator’s instructions exactly.

CAUTION: You MUST coordinate the IP addresses that you will use with your Avaya G700 Media Gateway with the IP addresses on the corporate LAN.  If you specify, for the media server component, an IP address that conflicts with another network endpoint, you can cause problems with traffic on the LAN. These problems can be extremely difficult to diagnose and resolve.

## Step 1a:  Get Stack Master Information

When you install multiple G700 Media Gateways in a stack, you designate one media gateway as the stack master. The stack master controls access to the other G700 Media Gateways. A stack master is also required when you have only one G700 Media Gateway. In this case (a stack of one), the stack master is the single media gateway.

Note: When you assign IP addresses for the media gateway and associated components, you may want to use a convention of consecutive IP addresses to create a consistent numbering scheme. This makes identification of components easier. For example:

My Company IP addresses:

subnet mask = 255.255.255.0

gateway = 149.83.124.250

For the example above, set IP addresses for the components in the G700 Media Gateway in consecutive order in the D portion (the fourth octet) of the IP address:

Stack Master (i960): 149.83.124.225

MGP: 149.83.124.226

VoIP: 149.83.124.227

| IP Address |  |
| --- | --- |
| Subnet Mask |  |

Note: The default subnet mask is 255.255.255.0. However, always ask the customer’s network administrator if this is the correct subnet mask IP address.

| VLAN Number |  |
| --- | --- |

Note: A VLAN is a defined network segment that allows users on that segment to have priority services in sharing information with each other. If the network is not using a VLAN, the VLAN number should be 1. If the network is using a VLAN, use the VLAN number provided by the LAN administrator.

### IP Routes

You must complete the Destination IP Address field, Destination Subnet Mask field, and the Gateway IP Address field for IP route 1. The other IP routes, 2 through 5, are optional.

| ##### Number | Destination IP Address | Destination Subnet Mask | Gateway IP Address |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 (optional) |  |  |  |
| 3 (optional) |  |  |  |
| 4 (optional) |  |  |  |
| 5 (optional) |  |  |  |

## Step 1b:  Get Media Gateway Information

The Avaya Installation Wizard automatically determines the number of media gateways in the stack.  Make a copy of this section for each media gateway in the configuration.

| Media Gateway Identifier |  |
| --- | --- |

Note: The media gateway identifier is the serial number of the G700 Media Gateway. This information is on a label on the back of the G700 Media Gateway.

### MGP

The Media Gateway Processor (MGP) is the processor on the motherboard of the media gateway. The MGP performs gateway functions and Voice Over IP (VOIP) resource assignments.

Enter any static IP addresses required to ensure delivery of information to certain endpoints in the network (for example, to access Ethernet switches or other G700 Media Gateways).

Note: When you assign IP addresses for the media gateway and associated components, you may want to use a convention of consecutive IP addresses to create a consistent numbering scheme. This makes identification of components easier. For example:

My Company IP addresses:

subnet mask = 255.255.255.0

gateway = 149.83.124.250

For the example above, set IP addresses for the components in the G700 in consecutive order in the D portion (the fourth octet) of the IP address:

S8300: 149.83.124.224

Stack Master (i960): 149.83.124.225

MGP: 149.83.124.226

VoIP: 149.83.124.227

| Destination IP Address |  |
| --- | --- |

Note: Enter the IP address of endpoint server is trying to reach.

| Subnet Mask |  |
| --- | --- |

Note: Enter the Subnet mask used by all endpoints.

| VLAN Number |  |
| --- | --- |

Note: A VLAN is a defined network segment that allows users on that segment to have priority services in sharing information with each other. If the network is not using a VLAN, the VLAN number should be 1.  If the network is using a VLAN, use the VLAN number provided by the LAN administrator.

| Number of IP routes to configure |  |
| --- | --- |

Note: An IP route specifies the network gateway to use to connect the local LAN segment to one or more distant networks. The default number of IP routes is one. You can enter a maximum of five routes. Enter the number of routes that the LAN administrator provides, and then enter the information for each route on the next page.

### IP Routes

The first column lists each IP route. You must complete the Destination IP Address field and the Network Gateway IP Address field for IP route 1. The other IP routes are optional.

| ##### Number | Destination IP Address | Network Gateway IP Address |
| --- | --- | --- |
| 1 |  |  |
| 2 (optional) |  |  |
| 3 (optional) |  |  |
| 4 (optional) |  |  |
| 5 (optional) |  |  |

### Media Gateway Controller List

The GIW is specifically for configuring a G700 Media Gateway that has an S8700 Media Server as its primary controller.  Therefore, your controller list will contain the following address types:

- Any CLAN board in the S8700 media server configuration. This type of board provides IP connectivity to the network of the S8700 primary controller.

- S8700 server 1. Use the specific S8700 server 1 IP address, not the active server address (also known as the alias address).

- S8700 server 2. A typical S8700 configuration always has a duplicate server installed. If server 2 is present, this specific IP address must also be entered. Again, use the specific S8700 IP address, not the active server “alias” address.

| Note: | If the endpoints of a G700 Media Gateway lose contact with the primary S8700 media server for any reason, they search their lists of addresses for another controller. |
| --- | --- |

| Controller 1 |  |
| --- | --- |
| Controller 2 |  |
| Controller 3 |  |
| Controller 4 |  |

### VoIP Modules – IP Addresses

Each media gateway has a Voice Over IP (VoIP) processor on the motherboard that manages VoIP calls. The location of the VoIP processor on the motherboard of the media gateway is indicated as V0. You can install an MM760 media module in any media gateway slot V1, V2, V3 or V4. The MM760 media module provides an additional 64 VoIP channels.

You must always assign an IP address for the VoIP processor. V0 in the Location column below indicates the VoIP processor on the motherboard of the media gateway. If you are installing additional VoIP modules in media gateway slots V1, V2, V3, or V4, assign IP addresses for each VoIP module.

Note: When you assign IP addresses for the media gateway and associated components, you may want to use a convention of consecutive IP addresses to create a consistent numbering scheme. This makes identification of components easier. For example:

My Company IP addresses:

subnet mask = 255.255.255.0

gateway = 149.83.124.250

For the example above, set IP addresses for the components in the G700 in consecutive order in the D portion (the fourth octet) of the IP address:

S8300: 149.83.124.224

Stack Master (i960): 149.83.124.225

MGP: 149.83.124.226

VoIP: 149.83.124.227

| Location | IP Address |
| --- | --- |
| V0 |  |
| V1 |  |
| V2 |  |
| V3 |  |
| V4 |  |

# Step 2:  Download the Avaya Gateway Installation Wizard

To download the GIW, connect your laptop to the Internet and go to http://support.avaya.com/avayaiw. Find the GIW executable file, doubleclick it, and follow the instructions.

# Step 3:  Set up a TFTP Server, If Necessary, and Download Firmware to a TFTP Server

See Chapter 4 in the Installation and Upgrades for G700 with S8300 or S8700, 555-234-100, Issue 3, for information on how to do this.

# Step 4:  Install Hardware for the G700 Media Gateway

See Chapter 4 in the Installation and Upgrades for G700 with S8300 or S8700, 555-234-100, Issue 3, for information on how to do this.

# Step 5:  Run the Avaya Gateway Installation Wizard

- With a direct connect cable, connect a COM port of your laptop to the Console port of the G700 Media Gateway. Remember which COM port you used.

- Locate the GIW executable file on your laptop, and doubleclick it.

- Use the data from this worksheet to complete the GIW.  Use the GIW Help button for additional information.

Note: Running the GIW automates the section “Configure the G700” in Chapter 4 in the Installation and Upgrades for G700 with S8300 or S8700.  However, you must complete Step 4 that follows, if applicable, without the aid of the GIW.

- Configure the X330 Expansion Module, if necessary.

# Step 6:  Prepare to Install the Firmware on the G700

See Chapter 4 in the Installation and Upgrades for G700 with S8300 or S8700, 555-234-100, Issue 3.

- Verify contents of the tftp directory

- Determine which firmware to install

# Step 7:  Install New Firmware on the G700 Media Gateway

See Chapter 4 in the Installation and Upgrades for G700 with S8300 or S8700, 555-234-100, Issue 3.

- Install firmware on the P330 stack processor

- Install firmware on the G700 media gateway processor

- Install firmware on the media modules

- Install firmware on other G700s in the stack or network, if any.

# Step 8:  Administer Communication Manager on the S8700 Primary Controller

See Chapter 4 in the Installation and Upgrades for G700 with S8300 or S8700, 555-234-100, Issue 3.

- Reboot the system

- Assign node names, if necessary

- Administer network regions

- Assign LSPs to network regions

- Administer IP interfaces

- Administer the LSP form

- Add media gateway

- Verify changes

- Enable announcements, if necessary

- Save translations

# Step 9:  Complete the Installation

To complete the installation, you must do the following as described in Chapter 4 in the Installation and Upgrades for G700 with S8300 or S8700, 555-234-100, Issue 3.

- Check planning documentation

- Connect and administer test endpoints

- Complete electrical installation Prepare to Install Firmware the G700.

# Step 10:  Register Warranty

Register the customer’s system as appropriate using the following information.

## Registration

| SAP Order Number |  |
| --- | --- |

| Installation Completion date |  |
| --- | --- |

Sale Type

Avaya Direct

Business Partner

## Customer Information 
(Required for all sales)

| Company Name |  |
| --- | --- |
| Street Address |  |
| City |  |
| State/Province |  |
| Zip/Postal Code |  |
| Country |  |
| Main Listed Telephone Number |  |
| Facility Location (FL, Optional) |  |

## Business Partner Information 
(Required only for Dealer sales)

| Company Name |  |
| --- | --- |
| Street Address |  |
| City |  |
| State/Province |  |
| Zip/Postal Code |  |
| Country |  |
| Main Listed Telephone Number |  |
| Facility Location (FL, Optional) |  |
| Distributor |  |

# Notes

All trademarks identified by the ® or TM are registered trademarks or trademarks, respectively, of Avaya Inc.  All other trademarks are the property of their respective owners.

Copyright  2003 Avaya Inc. All rights reserved